* TB 11-6625-316-12/1

| Technical Bulletin | HEADQUARTERS |
| --- | --- |
|  | DEPARTMENT OF THE ARMY |
| No. 11-6625-316-12/1 | Washington, D.C., 22 June 1966 |

TEST DATA FOR TEST SETS, ELECTRON TUBE TV-2/U, TV-2A/U, TV-2B/U, AND TV-2C/U

Note.  This bulletin supplements the data contained on the tube test data roll charts on the covers of Test Sets, Electron Tube TV-2/U, TV-2A/U, and TV-2B/U.

1.  Tube Test Data.  The tube test data provides information necessary to test some of the electron tubes that are not listes on the tube test data roll chart.  The column headings on this bulletin duplicate the column headings on the tube test data roll chart housing.  For a complete explanation of each column heading, refer to TM 11-6625-316-12.

2.  Abbreviations.  The letters “FS” and “RL” in the SIG VR column represent “full scale” and “red line” respectively.  The letters “CCW” in the BIAS column, under METER SETTING, represents “counter-clockwise.”

3.   Test Data.  The tube types appear in numerical and alphabetical order.  The test data listed from left to right in the order in which the coltrols should be set.

| Tube type | Test |  | Function |  |  | Selectors |  |  |  |  |  |  | Range |  |  |  |  |  |  |  |  |  |  | Meter Setting |  |  |  |  |  |  |  |  |  |  |  |  |  | Shunt |  |  |  |  |  | Press |  |  |  |  |  | Minimum |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | (L. to R.) |  |  |  |  |  |  | Filament |  |  | Bias |  |  | Pl SCR |  |  | Gm Sig | Plate |  |  |  |  | Bias |  |  | Screen |  |  |  | Sig VR |  |  |  |  |  |  |  |  | to test |  |  | Limits |  |  |  |  |  |  |
| 0C2 | VR | VR |  |  | 00 |  |  | 005 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 30 |  | MA |  |  | 0 |  |  | -------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 83 |  |  |  |  | Max. Max diff -4.5. |
| 0C2 | VR | VR |  |  | 00 |  |  | 005 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 5 |  | MA |  |  | 0 |  |  | -------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 68 |  |  |  |  | Min. Max diff -4.5. |
| 0G3 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | L |  |  | F |  | 9 |  | MA |  |  | 0 |  |  | -------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 88 |  |  |  |  | Max. Max diff -1.0. |
| 0G3 | VR | VR |  |  | 00 |  |  | 005 |  | 40 |  | OFF |  |  |  |  | 5 |  |  | L |  |  | F |  | 2 |  | MA |  |  | 0 |  |  | -------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 82 |  |  |  |  | Min. Max diff -1.0. |
| 0Z4A | EM | T |  |  | 00 |  |  | 030 |  | 80 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | A |  | 200 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 90 |  |  |  |  | Diode #1. |
| 0Z4A | EM | T |  |  | 00 |  |  | 050 |  | 80 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | A |  | 200 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 0Z4G | EM | T |  |  | 00 |  |  | 030 |  | 80 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | A |  | 200 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 0Z4G | EM | T |  |  | 00 |  |  | 050 |  | 80 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | A |  | 200 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 1AB6 | GM | T |  |  | 17 |  |  | 435 |  | 02 |  | 1.5 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 3.5 |  |  | 45 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1AC5 | GM | T |  |  | 45 |  |  | 278 |  | 00 |  | 1.1 |  |  |  |  | 10 |  |  | N |  |  | E |  | 67.5 |  |  |  |  | 4.5 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1AC6 | GM | T |  |  | 17 |  |  | 432 |  | 65 |  | 1.5 |  |  |  |  | 5 |  |  | Q |  |  | D |  | 45 |  |  |  |  | 2.5 |  |  | 45 |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1AD5 | GM | T |  |  | 45 |  |  | 278 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | N |  |  | E |  | 67.5 |  |  |  |  | 2.5 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1AF4 | GM | T |  |  | 17 |  |  | 623 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 1.2 |  |  | 90 |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1AF5 | GM | T |  |  | 17 |  |  | 654 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | E |  | 90 |  |  |  |  | 1.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.4. |
| 1AF5 | EM | T |  |  | 17 |  |  | 030 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 1.4. |
| 1AG4 | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | D |  | 45 |  |  |  |  | 3.5 |  |  | 45 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| 1AG5 | GM | T |  |  | 46 |  |  | 512 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 1AG5 | EM | T |  |  | 46 |  |  | 030 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 26 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. Adj fil to 1.25. |
| 1AJ4 | GM | T |  |  | 71 |  |  | 623 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 1.6 |  |  | 90 |  |  |  | RL |  |  |  |  | 96 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1AJ5 | GM | T |  |  | 46 |  |  | 512 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 1AJ5 | EM | T |  |  | 46 |  |  | 030 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. Adj fil to 1.2. |
| 1AK4 | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| 1AK5 | GM | T |  |  | 46 |  |  | 512 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 1AK5 | EM | T |  |  | 46 |  |  | 030 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 26 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. Adj fil to 1.25. |
| 1C3 | GM | T |  |  | 17 |  |  | 420 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | E |  | 90 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1DN5 | GM | T |  |  | 17 |  |  | 623 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | E |  | 90 |  |  |  |  | 4.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.4. |
| 1DN5 | EM | T |  |  | 17 |  |  | 040 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20V |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 1.4. |
| 1E3 | GM | T |  |  | 45 |  |  | 180 |  | 00 |  | 1.1 |  |  |  |  | 10 |  |  | G |  |  | C |  | 150 |  |  |  |  | 3.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 59 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| 1H2 | EM | VR |  |  | 12 |  |  | 00A |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | K |  |  | F |  | (#) |  |  |  |  | 0 |  |  | (#) |  |  |  | CW |  |  |  |  | 50 |  |  |  | P5 |  |  | (* ) |  |  |  |  | Adj fil to 1.4. |
| 1M3 | ER | T |  |  | 45 |  |  | 180 |  | 00 |  | 1.5 |  |  |  |  | 50 |  |  | P |  |  | A |  | 100 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  |  |
| 1S2A | EM | VR |  |  | 12 |  |  | 00A |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | L |  |  | C |  | ----- |  |  |  |  | 0 |  |  | 70 |  |  |  | FS |  |  |  |  | 45 |  |  |  | P5 |  |  | ----------- |  |  |  |  | OK under 75 (Adj fil to 1.4). |
| 1T6 | GM | T |  |  | 45 |  |  | 318 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | N |  |  | E |  | 67.5 |  |  |  |  | .5 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 50 |  |  |  |  | Diode sect. |
| 1T6 | EM | T |  |  | 45 |  |  | 060 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | E |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 48 |  |  |  | P2 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1U6 | GM | T |  |  | 17 |  |  | 453 |  | 26 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | E |  | 65 |  |  |  |  | 1.0 |  |  | 65 |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | ----------- |  |  |  |  |  |
| 1V6 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 1V6 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 3.5 |  |  | 45 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 1.25. |
| 1W4 | GM | T |  |  | 45 |  |  | 782 |  | 19 |  | 1.5 |  |  |  |  | 50 |  |  | N |  |  | D |  | 90 |  |  |  |  | 9.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.4. |
| 1Z2 | EM | T |  |  | 17 |  |  | 0B0 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | G |  |  | A |  | 225 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 72 |  |  |  |  | Adj fil to 1.25. |
| 2AF4 | GM | T |  |  | 34 |  |  | 210 |  | 50 |  | 2.5 |  |  |  |  | C |  |  | N |  |  | B |  | 80 |  |  |  |  | CCW |  |  | 0 |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.35. |
| 2B3 | EM | T |  |  | 27 |  |  | 0A0 |  | 00 |  | 2.0 |  |  |  |  | 5 |  |  | N |  |  | A |  | 100 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P2 |  |  | 50 |  |  |  |  | Adj fil to 1.75. |
| 2BN4 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 2.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.1. |
| 2BN4A | GM | T |  |  | 43 |  |  | 250 |  | 10 |  | 2.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.35. |
| 2CW4 | GM | T |  |  | 31 |  |  | 420 |  | 80 |  | 2.0 |  |  |  |  | 5 |  |  | N |  |  | B |  | 70 |  |  |  |  | .2 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-127. |
| 2CY5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 2.5 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.4. |
| 2D21W | TH | VR |  |  | 34 |  |  | 106 |  | 27 |  | 6.3 |  |  |  |  | 10 |  |  | L |  |  | F |  | ----- |  |  |  |  | * 10 |  |  | 120 |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | Bias |  |  |  |  | Volts 1.5 min, 3.0 max. |
| 2E5 | ER | T |  |  | 16 |  |  | 040 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | K |  |  | F |  | 200 |  |  |  |  | 0 |  |  | 200 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye open. |
| 2E5 | ER | T |  |  | 16 |  |  | 042 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | K |  |  | F |  | 200 |  |  |  |  | 0 |  |  | 200 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye closed. |
| 2E5 | EM | T |  |  | 16 |  |  | 342 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P2,P3 |  |  | 50 |  |  |  |  | Triode sect. |
| 2E36 | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.25 |  |  | 45 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 2E41 | GM | T |  |  | 46 |  |  | 512 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 2E41 | EM | T |  |  | 46 |  |  | 030 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 26 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 1.25. |
| 2E42 | GM | T |  |  | 46 |  |  | 512 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.0 |  |  | 45 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 1.25. |
| 2E42 | EM | T |  |  | 46 |  |  | 039 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 26 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 1.25. |
| 2EA5 | GM | T |  |  | 43 |  |  | 156 |  | 20 |  | 2.5 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 1.0 |  |  | 140 |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.3. |
| 2EN5 | EM | T |  |  | 43 |  |  | 070 |  | 56 |  | 2.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 2EN5 | EM | T |  |  | 43 |  |  | 020 |  | 56 |  | 2.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 2ER5 | GM | T |  |  | 43 |  |  | 256 |  | 70 |  | 2.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 200 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.1. |
| 2ES5 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 2.5 |  |  |  |  | 5 |  |  | G |  |  | B |  | 200 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 63 |  |  |  |  | Adj fil to 2.35. |
| 2EV5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 2.5 |  |  |  |  | 5 |  |  | M |  |  | B |  | 195 |  |  |  |  | 1.2 |  |  | 80 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 63 |  |  |  |  | Adj fil to 2.4. |
| 2FH5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 2.5 |  |  |  |  | 5 |  |  | M |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 2FQ5 | GM | T |  |  | 43 |  |  | 250 |  | 70 |  | 2.5 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.3. |
| 2FS5 | GM | T |  |  | 34 |  |  | 156 |  | 70 |  | 2.5 |  |  |  |  | 5 |  |  | J |  |  | B |  | 245 |  |  |  |  | .2 |  |  | 135 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.4. |
| 2FY5 | GM | T |  |  | 34 |  |  | 250 |  | 16 |  | 2.5 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.4. |
| 2G22 | GM | T |  |  | 47 |  |  | 325 |  | 51 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 3.0 |  |  | 43 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Heptode sect. Adj fil to 1.25. |
| 2G22 | GM | T |  |  | 47 |  |  | 310 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 40 |  |  |  |  | 3.0 |  |  | 40 |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 1.25. |
| 2GK5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 2.5 |  |  |  |  | 5 |  |  | K |  |  | A |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.3. |
| 2T4 | GM | T |  |  | 34 |  |  | 210 |  | 50 |  | 2.5 |  |  |  |  | C |  |  | N |  |  | B |  | 80 |  |  |  |  | CCW |  |  | 0 |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.35. |
| 2V2 | EM | T |  |  | 27 |  |  | 0A0 |  | 00 |  | 2.5 |  |  |  |  | 5 |  |  | G |  |  | A |  | 225 |  |  |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  |  |
| 3A2 | EM | T |  |  | 12 |  |  | 0A0 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | G |  |  | A |  | 150 |  |  |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Adj fil to 3.15. |
| 3A3 | EM | T |  |  | 27 |  |  | 0A0 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | G |  |  | A |  | 225 |  |  |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Adj fil to 3.15. |
| 3A4 | GM | T |  |  | 17 |  |  | 423 |  | 00 |  | 3.0 |  |  |  |  | 10 |  |  | N |  |  | D |  | 75 |  |  |  |  | 5.7 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3AF4A | GM | T |  |  | 34 |  |  | 210 |  | 50 |  | 3.0 |  |  |  |  | C |  |  | N |  |  | B |  | 80 |  |  |  |  | CCW |  |  | 0 |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.2. |
| 3AL5 | EM | T |  |  | 34 |  |  | 070 |  | 10 |  | 3.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 3.15. |
| 3AL5 | EM | T |  |  | 34 |  |  | 020 |  | 50 |  | 3.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 3.15. |
| 3AU6 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 3.0 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3AV6 | GM | T |  |  | 34 |  |  | 170 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 3.15. |
| 3AV6 | EM | T |  |  | 34 |  |  | 060 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode #1. Adj fil to 3.15. |
| 3AV6 | EM | T |  |  | 34 |  |  | 050 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode #2. Adj fil to 3.15. |
| 3B2 | EM | T |  |  | 27 |  |  | 0A0 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | G |  |  | A |  | 225 |  |  |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P2 |  |  | 50 |  |  |  |  | Adj fil to 3.15. |
| 3B4 | GM | T |  |  | 45 |  |  | 371 |  | 00 |  | 2.5 |  |  |  |  | 50 |  |  | H |  |  | D |  | 200 |  |  |  |  | 25 |  |  | 150 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 50 |  |  |  |  |  |
| 3BA6 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 3.0 |  |  |  |  | A |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3BC5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 3.0 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3BE6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | A |  | 110 |  |  |  |  | 1.0 |  |  | 110 |  |  |  | RL |  |  |  |  | 98 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3BN4 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 3.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3BN4A | GM | T |  |  | 43 |  |  | 250 |  | 10 |  | 3.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 3BN6 | GM | T |  |  | 34 |  |  | 275 |  | 16 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 75 |  |  |  |  | 2.0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3BU8 | GM | T |  |  | 45 |  |  | 782 |  | 19 |  | 3.0 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. Adj fil to 3.15. |
| 3BU8 | GM | T |  |  | 45 |  |  | 732 |  | 16 |  | 3.0 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. Adj fil to 3.15. |
| 3BX6 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 3.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 170 |  |  |  |  | 2.0 |  |  | 170 |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.4. |
| 3BY6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 3.0 |  |  |  |  | 5 |  |  | P |  |  | C |  | 100 |  |  |  |  | 2.0 |  |  | 50 |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3BZ6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 3.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3C2 | EM | T |  |  | 27 |  |  | 0A0 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | G |  |  | A |  | 150 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P2 |  |  | 50 |  |  |  |  | Adj fil to 3.15. |
| 3C4 | GM | T |  |  | 17 |  |  | 623 |  | 00 |  | 2.5 |  |  |  |  | 10 |  |  | N |  |  | C |  | 85 |  |  |  |  | 5.2 |  |  | 85 |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3CB6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 3.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3CE5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 3.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3CF6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 3.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3CS6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | C |  | 80 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3CY5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 1 |  |  | 80 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.9. |
| 3D6/1299 | GM | T |  |  | 81 |  |  | 623 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.5 |  |  | 90 |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3DK6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 3.0 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3DT6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 3.0 |  |  |  |  | E |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.15. |
| 3E5 | GM | T |  |  | 17 |  |  | 623 |  | 00 |  | 2.5 |  |  |  |  | 10 |  |  | N |  |  | D |  | 90 |  |  |  |  | 8.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 3E6 | GM | T |  |  | 18 |  |  | 623 |  | 04 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 1.8 |  |  | 90 |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 3E29 | GM | T |  |  | 57 |  |  | 6B3 |  | 40 |  | 6.3 |  |  |  |  | 50 |  |  | H |  |  | B |  | 220 |  |  |  |  | 10 |  |  | 155 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 60 |  |  |  |  | Pentode #1. |
| 3E29 | GM | T |  |  | 51 |  |  | 2A3 |  | 40 |  | 6.3 |  |  |  |  | 50 |  |  | H |  |  | B |  | 220 |  |  |  |  | 10 |  |  | 155 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 60 |  |  |  |  | Pentode #2. |
| 3EA5 | GM | T |  |  | 43 |  |  | 156 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 1.0 |  |  | 140 |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 3ER5 | GM | T |  |  | 43 |  |  | 256 |  | 70 |  | 3.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 200 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3EV5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 3.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 195 |  |  |  |  | 1.2 |  |  | 80 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 63 |  |  |  |  | Adj fil to 2.9. |
| 3FH5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 3.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 3FQ5 | GM | T |  |  | 43 |  |  | 250 |  | 70 |  | 3.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3FV5 | GM | T |  |  | 34 |  |  | 250 |  | 16 |  | 3.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.1. |
| 3GK5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 3.0 |  |  |  |  | 5 |  |  | K |  |  | A |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3GS8 | GM | T |  |  | 54 |  |  | 782 |  | 19 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. Adj fil to 3.15. |
| 3GS8 | GM | T |  |  | 54 |  |  | 732 |  | 16 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. Adj fil to 3.15. |
| 3LE4 | GM | T |  |  | 81 |  |  | 623 |  | 00 |  | 3.0 |  |  |  |  | 10 |  |  | N |  |  | D |  | 90 |  |  |  |  | 9 |  |  | 90 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 60 |  |  |  |  | Adj fil to 2.8. |
| 3LF4 | GM | T |  |  | 81 |  |  | 623 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.5 |  |  | 90 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3D4 | GM | T |  |  | 71 |  |  | 324 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.5 |  |  | 90 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3Q5GT | GM | T |  |  | 72 |  |  | 534 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.5 |  |  | 90 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3S4 | GM | T |  |  | 17 |  |  | 324 |  | 00 |  | 3.0 |  |  |  |  | 10 |  |  | N |  |  | D |  | 75 |  |  |  |  | 7 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 2.8. |
| 3V4 | GM | T |  |  | 17 |  |  | 623 |  | 00 |  | 3.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.5 |  |  | 90 |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 67 |  |  |  |  | Adj fil to 2.8. |
| 4AU6 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 4.2 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4AV6 | GM | T |  |  | 34 |  |  | 170 |  | 20 |  | 4.2 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 4AV6 | EM | T |  |  | 34 |  |  | 060 |  | 20 |  | 4.2 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode #1. |
| 4AV6 | EM | T |  |  | 34 |  |  | 050 |  | 20 |  | 4.2 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode #2. |
| 4BA6 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 4.2 |  |  |  |  | A |  |  | M |  |  | C |  | 220 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BC5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 4.2 |  |  |  |  | C |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BC8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.2. |
| 4BC8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.2. |
| 4BE6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | A |  | 110 |  |  |  |  | 1.0 |  |  | 110 |  |  |  | RL |  |  |  |  | 98 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BL8 | GM | T |  |  | 54 |  |  | 263 |  | 71 |  | 4.2 |  |  |  |  | 5 |  |  | K |  |  | B |  | 170 |  |  |  |  | 2.1 |  |  | 170 |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.6. |
| 4BL8 | GM | T |  |  | 54 |  |  | 910 |  | 86 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.6. |
| 4BN4 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BN6 | GM | T |  |  | 34 |  |  | 275 |  | 16 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | D |  | 75 |  |  |  |  | 2.0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BQ7A | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.2. |
| 4BQ7A | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.2. |
| 4BS8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.2. |
| 4BS8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.2. |
| 4BU8 | GM | T |  |  | 45 |  |  | 782 |  | 19 |  | 4.2 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. |
| 4BU8 | GM | T |  |  | 45 |  |  | 732 |  | 16 |  | 4.2 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. |
| 4BX8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 4.2 |  |  |  |  | 5 |  |  | P |  |  | B |  | 65 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.5. |
| 4BX8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 4.2 |  |  |  |  | 5 |  |  | P |  |  | B |  | 65 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.5. |
| 4BZ6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 4.2 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4BZ7 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.2. |
| 4BZ7 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 4.2 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.2. |
| 4BZ8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 4.2 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 4BZ8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 4.2 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 4CB6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 4.2 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4CE5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 4.2 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4CS6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | C |  | 80 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4CY5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 4.2 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.5. |
| 4DE6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 4.2 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4DT6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 4.2 |  |  |  |  | E |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4ES8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 4ES8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 4EW6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 4.2 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 4GS8 | GM | T |  |  | 54 |  |  | 782 |  | 19 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. |
| 4GS8 | GM | T |  |  | 54 |  |  | 732 |  | 16 |  | 4.2 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. |
| 5AM8 | GM | T |  |  | 45 |  |  | 263 |  | 19 |  | 5.0 |  |  |  |  | C |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5AM8 | EM | T |  |  | 45 |  |  | 080 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 4.7. |
| 5AM8 | GM | T |  |  | 45 |  |  | 867 |  | 91 |  | 5.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5AN8 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 5.0 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5AQ5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 5.0 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.7. |
| 5AS8 | GM | T |  |  | 45 |  |  | 291 |  | 37 |  | 5.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5AS8 | EM | T |  |  | 45 |  |  | 060 |  | 87 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 18 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. Adj fil to 4.7. |
| 5AT8 | GM | T |  |  | 45 |  |  | 967 |  | 38 |  | 5.0 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5AT8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Diode sect. Adj fil to 4.7. |
| 5AV8 | GM | T |  |  | 45 |  |  | 698 |  | 70 |  | 5.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5AV8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 5.0 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5B8 | GM | T |  |  | 45 |  |  | 698 |  | 71 |  | 5.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5B8 | GM | T |  |  | 45 |  |  | 230 |  | 19 |  | 5.0 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5BE8 | GM | T |  |  | 45 |  |  | 967 |  | 83 |  | 5.0 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5BE8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5BK7A | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 5.0 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.7. |
| 5BK7A | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 5.0 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.7. |
| 5BQ7A | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 5.6. |
| 5BQ7A | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 5.6. |
| 5BR8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 5.0 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5BR8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5BS8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 5.6. |
| 5BS8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 5.6. |
| 5BT8 | GM | T |  |  | 45 |  |  | 867 |  | 90 |  | 5.0 |  |  |  |  | C |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5BT8 | EM | T |  |  | 45 |  |  | 010 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 4.7. |
| 5BT8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 4.7. |
| 5BW8 | GM | T |  |  | 54 |  |  | 698 |  | 70 |  | 5.0 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5BW8 | EM | T |  |  | 54 |  |  | 030 |  | 21 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 4.7. |
| 5BW8 | EM | T |  |  | 54 |  |  | 010 |  | 23 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 4.7. |
| 5BZ7 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 5.6. |
| 5BZ7 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 5.0 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 5.6. |
| 5CG8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 5.0 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5CG8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5CL8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. Adj fil to 4.7. |
| 5CL8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5CM6 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 5.0 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.7. |
| 5CM8 | GM | T |  |  | 45 |  |  | 267 |  | 30 |  | 5.0 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5CM8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5CQ8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. Adj fil to 4.7. |
| 5CQ8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 5.0 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5CZ5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 5.0 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 14.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 39 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.7. |
| 5DH8 | GM | T |  |  | 45 |  |  | 967 |  | 83 |  | 5.0 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 5.2. |
| 5DH8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | E |  |  | J |  |  | C |  | 200 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 5.2. |
| 5EA8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5EA8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 5.0 |  |  |  |  | A |  |  | K |  |  | B |  | 150 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sec. Adj fil to 4.7. |
| 5EH8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sec. Adj fil to 4.7. |
| 5EH8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 5.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5ES8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 5.6. |
| 5ES8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 5.6. |
| 5EU8 | GM | T |  |  | 45 |  |  | 719 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5EU8 | GM | T |  |  | 45 |  |  | 230 |  | 60 |  | 5.0 |  |  |  |  | A |  |  | K |  |  | B |  | 150 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sec. Adj fil to 4.7. |
| 5EW6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 5.0 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 5.6. |
| 5FG7 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5FG7 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.1 |  |  | -------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5FV8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5FV8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5GH8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5GH8 | GM | T |  |  | 45 |  |  | 110 |  | 80 |  | 5.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5J6 | GM | T |  |  | 34 |  |  | 520 |  | 76 |  | 5.0 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 4.7. |
| 5J6 | GM | T |  |  | 34 |  |  | 610 |  | 75 |  | 5.0 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 4.7. |
| 5R4G4 | EM | T |  |  | 28 |  |  | 060 |  | 00 |  | 5.0 |  |  |  |  | 5 |  |  | R |  |  | A |  | 35 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P2 |  |  | 63 |  |  |  |  | Diode #1. |
| 5R4G4 | EM | T |  |  | 28 |  |  | 040 |  | 00 |  | 5.0 |  |  |  |  | 5 |  |  | R |  |  | A |  | 35 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P2 |  |  | 68 |  |  |  |  | Diode #2. |
| 5T8 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 5T8 | EM | T |  |  | 45 |  |  | 060 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 5T8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 5T8 | EM | T |  |  | 45 |  |  | 010 |  | 70 |  | 5.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 5U8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 5.0 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5U8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 5.0 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 5V6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 5.0 |  |  |  |  | 50 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 59 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.7. |
| 5X8 | GM | T |  |  | 45 |  |  | 798 |  | 61 |  | 5.0 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 4.7. |
| 5X8 | GM | T |  |  | 45 |  |  | 230 |  | 60 |  | 5.0 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 4.7. |
| 6AB8 | GM | T |  |  | 45 |  |  | 968 |  | 37 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | C |  | 170 |  |  |  |  | 6.5 |  |  | 170 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AB8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AJ8 | GM | T |  |  | 45 |  |  | 261 |  | 37 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | C |  | 200 |  |  |  |  | 1.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  | Heptode sect. |
| 6AJ8 | GM | T |  |  | 45 |  |  | 980 |  | 36 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | C |  | 150 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AK8 | GM | T |  |  | 45 |  |  | 890 |  | 76 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AK8 | EM | T |  |  | 45 |  |  | 060 |  | 79 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6AK8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6AK8 | EM | T |  |  | 45 |  |  | 010 |  | 79 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 6AN8 | GM | T |  |  | 45 |  |  | 867 |  | 91 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AN8 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AQ4 | GM | T |  |  | 43 |  |  | 170 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6AQ5 | GM | T |  |  | 34 |  |  | 156 |  | 22 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 230 |  |  |  |  | 12.5 |  |  | 225 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 58 |  |  |  |  | Short in Z. |
| 6AQ8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6AQ8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6AR5 | GM | T |  |  | 34 |  |  | 156 |  | 22 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | D |  | 230 |  |  |  |  | 18 |  |  | 225 |  |  |  | RL |  |  |  |  | 36 |  |  |  | P4 |  |  | 65 |  |  |  |  | Short in Z. |
| 6AR6/6098 | GM | T |  |  | 68 |  |  | 735 |  | 10 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 230 |  |  |  |  | 22 |  |  | 225 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6AR8 | GM | T |  |  | 45 |  |  | 693 |  | 72 |  | 6.3 |  |  |  |  | E |  |  | G |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 250 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Plate #1. |
| 6AR8 | GM | T |  |  | 45 |  |  | 683 |  | 71 |  | 6.3 |  |  |  |  | E |  |  | G |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 250 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Plate #2. |
| 6AS8 | GM | T |  |  | 45 |  |  | 291 |  | 37 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pemtode sect. |
| 6AS8 | EM | T |  |  | 45 |  |  | 060 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 18 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 6AT8 | GM | T |  |  | 45 |  |  | 967 |  | 38 |  | 6.3 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AT8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AU7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Trriode #1. |
| 6AU7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6AU8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AU8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | C |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AW7 | GM | T |  |  | 78 |  |  | 260 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | D |  | 150 |  |  |  |  | 1.9 |  |  | -------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AW7 | EM | T |  |  | 78 |  |  | 230 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 22 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6AW7 | EM | T |  |  | 78 |  |  | 240 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 22 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6AW8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AW8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | H |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AX7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 225 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6AX7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 225 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6AX8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AX8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6AZ5 | EM | T |  |  | 36 |  |  | 080 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6AZ5 | EM | T |  |  | 36 |  |  | 010 |  | 24 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6AZ6 | EM | T |  |  | 36 |  |  | 020 |  | 48 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6AZ6 | EM | T |  |  | 36 |  |  | 070 |  | 58 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6AZ8 | GM | T |  |  | 45 |  |  | 612 |  | 30 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6AZ8 | GM | T |  |  | 45 |  |  | 980 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BA5 | GM | T |  |  | 36 |  |  | 157 |  | 80 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BA8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 51 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BA8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BC4 | GM | T |  |  | 45 |  |  | 210 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | K |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BC8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6BC8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6BE7 | GM | T |  |  | 45 |  |  | 716 |  | 82 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 70 |  |  |  |  | 2.0 |  |  | 70 |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BE8 | GM | T |  |  | 45 |  |  | 967 |  | 83 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BE8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BF5 | GM | T |  |  | 34 |  |  | 756 |  | 22 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 125 |  |  |  |  | 6.8 |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 60 |  |  |  |  | Short in Z. |
| 6BG7 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6BG7 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 00 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6BH8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | A |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BH8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | M |  |  | C |  | 150 |  |  |  |  | 5.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BJ7 | EM | T |  |  | 45 |  |  | 080 |  | 93 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BJ7 | EM | T |  |  | 45 |  |  | 060 |  | 73 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BJ7 | EM | T |  |  | 45 |  |  | 020 |  | 13 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 6BJ8 | GM | T |  |  | 45 |  |  | 870 |  | 90 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 9.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BJ8 | EM | T |  |  | 45 |  |  | 060 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BJ8 | EM | T |  |  | 45 |  |  | 010 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BL8 | GM | T |  |  | 54 |  |  | 263 |  | 71 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 170 |  |  |  |  | 2.1 |  |  | 170 |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BL8 | GM | T |  |  | 54 |  |  | 910 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BM8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BM8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 0. |  |  | -------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BN4 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BN4A | GM | T |  |  | 43 |  |  | 250 |  | 10 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BN8 | GM | T |  |  | 45 |  |  | 870 |  | 90 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 100 |  |  |  |  | 1. |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BN8 | EM | T |  |  | 45 |  |  | 060 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BN8 | EM | T |  |  | 45 |  |  | 010 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BR5 | ER | T |  |  | 45 |  |  | 197 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 200 |  |  |  |  | 0 |  |  | 200 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ---------- |  |  |  |  | Eye open. |
| 6BR5 | ER | T |  |  | 45 |  |  | 190 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 200 |  |  |  |  | 0 |  |  | 200 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ---------- |  |  |  |  | Eye closed. |
| 6BR8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BR8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | A |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BS7 | GM | T |  |  | 45 |  |  | A78 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 210 |  |  |  |  | 3.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BS8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6BS8 | GM | T |  |  | 45 |  |  | 210 |  | 80 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6BT8 | GM | T |  |  | 45 |  |  | 867 |  | 90 |  | 6.3 |  |  |  |  | C |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BT8 | EM | T |  |  | 45 |  |  | 010 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BT8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BU8 | GM | T |  |  | 45 |  |  | 782 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. |
| 6BU8 | GM | T |  |  | 45 |  |  | 732 |  | 16 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. |
| 6BV8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | E |  |  | G |  |  | B |  | 200 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6BV8 | EM | T |  |  | 45 |  |  | 00 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 19 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BV8 | EM | T |  |  | 45 |  |  | 060 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 19 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BW8 | GM | T |  |  | 45 |  |  | 698 |  | 70 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BW8 | EM | T |  |  | 45 |  |  | 030 |  | 21 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6BW8 | EM | T |  |  | 45 |  |  | 010 |  | 23 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6BX6 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 170 |  |  |  |  | 2.0 |  |  | 170 |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BX8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 6.3 |  |  |  |  | 5 |  |  | P |  |  | B |  | 65 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6BX8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | P |  |  | B |  | 65 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6BY6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | P |  |  | C |  | 100 |  |  |  |  | 2.0 |  |  | 50 |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BY7 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 225 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BY8 | GM | T |  |  | 45 |  |  | 178 |  | 92 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6BY8 | EM | T |  |  | 45 |  |  | 060 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode Sect. |
| 6BZ6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BZ8 | GM | T |  |  | 45 |  |  | 760 |  | 86 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6BZ8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CA5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | B |  | 125 |  |  |  |  | 4.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CA7 | GM | T |  |  | 27 |  |  | 534 |  | 81 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 13.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6BC5 | GM | T |  |  | 27 |  |  | 4A1 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CD7 | ER | T |  |  | 72 |  |  | 450 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | E |  | 200 |  |  |  |  | ------ |  |  | -------- |  |  |  | -------- |  |  |  |  | 0 |  |  |  | P5 |  |  | ----------- |  |  |  |  |  |
| 6CE5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CF6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CG8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 6.3 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CG8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CH7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6CH7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | D |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CH8 | GM | T |  |  | 45 |  |  | 723 |  | 60 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CH8 | GM | T |  |  | 45 |  |  | 890 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CK4 | GM | T |  |  | 72 |  |  | 150 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 28.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CL5 | GM | T |  |  | 72 |  |  | 5A1 |  | 60 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 175 |  |  |  |  | 40.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CL5 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 7CL8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CM4 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 175 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CM5 | GM | T |  |  | 72 |  |  | 5A4 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 8.2 |  |  | 100 |  |  |  | RL |  |  |  |  | 22 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CM6 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CM7 | GM | T |  |  | 45 |  |  | 760 |  | 30 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 7.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6CM7 | GM | T |  |  | 45 |  |  | 810 |  | 90 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 36 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CM8 | GM | T |  |  | 45 |  |  | 267 |  | 30 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CM8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CN6 | GM | T |  |  | 72 |  |  | 5A4 |  | 91 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 22 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CN7 | GM | T |  |  | 45 |  |  | 780 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triod sect. |
| 6CN7 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6CN7 | EM | T |  |  | 45 |  |  | 010 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6CQ8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 150 |  |  |  |  | 1 |  |  | 125 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 6CQ8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CR6 | GM | T |  |  | 34 |  |  | 756 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CR6 | EM | T |  |  | 34 |  |  | 020 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 6CR8 | GM | T |  |  | 45 |  |  | 267 |  | 38 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 0.7 |  |  | 125 |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 6CR8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 125 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6CS6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 80 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CS7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6CS7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 10.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CU5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 120 |  |  |  |  | .0 |  |  | 110 |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CU6 | GM | T |  |  | 72 |  |  | 5A4 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CU8 | GM | T |  |  | 45 |  |  | 723 |  | 60 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CU8 | GM | T |  |  | 45 |  |  | 890 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 6.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CW4 | GM | T |  |  | 31 |  |  | 420 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 70 |  |  |  |  | 0.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-127. |
| 6CW5 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 170 |  |  |  |  | 12.0 |  |  | 170 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6CW7 | GM | T |  |  | 45 |  |  | 690 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | P |  |  | B |  | 90 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6CW7 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | P |  |  | B |  | 90 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CX8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6CX8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | C |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6CY7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6CY7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | B |  | 90 |  |  |  |  | CCW |  |  | -------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6CZ5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 14.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 39 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DA6 | GM | T |  |  | 45 |  |  | 278 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 225 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DA7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DA7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | B |  | 150 |  |  |  |  | 17.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DB5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DB6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 150 |  |  |  |  | 1.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DC8 | GM | T |  |  | 45 |  |  | 261 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 220 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6DC8 | EM | T |  |  | 45 |  |  | 080 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6DC8 | EM | T |  |  | 45 |  |  | 070 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6DE6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DE7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | C |  | 225 |  |  |  |  | 12.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DE7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 150 |  |  |  |  | 26.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DG6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DG7 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | C |  | 175 |  |  |  |  | CCW |  |  | 80 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DK6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DJ8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.3 |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DJ8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.3 |  |  | -------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DL5 | GM | T |  |  | 43 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 9.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6DN6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 225 |  |  |  |  | 18.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DN7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DN7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | B |  | 250 |  |  |  |  | 9.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DQ5 | GM | T |  |  | 72 |  |  | 1A4 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | B |  | 200 |  |  |  |  | 25.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DQ6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DR7 | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DR7 | GM | T |  |  | 54 |  |  | 210 |  | 90 |  | 6.3 |  |  |  |  | 50 |  |  | M |  |  | C |  | 150 |  |  |  |  | 17.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DS5 | GM | T |  |  | 24 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 200 |  |  |  |  | 7.5 |  |  | 200 |  |  |  | RL |  |  |  |  | 84 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DT5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 16.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DT6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | E |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6DT8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6DT8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6DW5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | H |  |  | C |  | 210 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 60 |  |  |  |  |  |
| 6DX8 | GM | T |  |  | 45 |  |  | 869 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | B |  | 200 |  |  |  |  | 2.8 |  |  | 200 |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6DX8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 200 |  |  |  |  | 2.1 |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6DZ7 | GM | T |  |  | 27 |  |  | 564 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.3 |  |  | 250 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode #1. |
| 6DZ7 | GM | T |  |  | 27 |  |  | 134 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.3 |  |  | 250 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode #2. |
| 6EA5 | GM | T |  |  | 43 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 1.0 |  |  | 140 |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EA7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 6EA7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | L |  |  | B |  | 175 |  |  |  |  | 25.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 62 |  |  |  |  | Triode #2. |
| 6EA8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6EA8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | K |  |  | B |  | 150 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tiode sect. |
| 6EB5 | EM | T |  |  | 34 |  |  | 070 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 27 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 6EB5 | EM | T |  |  | 34 |  |  | 020 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 27 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 6EB8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6EB8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6EH5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | K |  |  | B |  | 115 |  |  |  |  | 4.8 |  |  | 110 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EH7 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | A |  | 200 |  |  |  |  | 2.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EH8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6EH8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6EJ7 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | A |  | 200 |  |  |  |  | 2.5 |  |  | 200 |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EM5 | GM | T |  |  | 54 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 18.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EM7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 6EM7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | L |  |  | B |  | 150 |  |  |  |  | 20.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. |
| 6EQ7 | GM | T |  |  | 45 |  |  | 276 |  | 31 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | .6 |  |  | 100 |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6EQ7 | EM | T |  |  | 45 |  |  | 080 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 6ER5 | GM | T |  |  | 43 |  |  | 256 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 200 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6ES5 | GM | T |  |  | 34 |  |  | 250 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 200 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6ES8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6ES8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 90 |  |  |  |  | 1.7 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6ET7 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | B |  | 200 |  |  |  |  | 2.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 29 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6ET7 | EM | T |  |  | 45 |  |  | 030 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 6ET7 | EM | T |  |  | 45 |  |  | 020 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 6EU7 | GM | T |  |  | 21 |  |  | 870 |  | 90 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 200 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6EU7 | GM | T |  |  | 21 |  |  | 560 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 200 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6EU8 | GM | T |  |  | 45 |  |  | 719 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6EU8 | GM | T |  |  | 45 |  |  | 230 |  | 60 |  | 6.3 |  |  |  |  | A |  |  | K |  |  | B |  | 150 |  |  |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6EV5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 195 |  |  |  |  | 1.2 |  |  | 80 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6EV7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6EV7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6EW6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6EW7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | C |  | 250 |  |  |  |  | 11.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6EW7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 150 |  |  |  |  | 17.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6EX6 | GM | T |  |  | 72 |  |  | 5A8 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tubes showling shorts: Retest 72 5A1 30. |
| 6EY6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 17.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6EZ5 | GM | T |  |  | 72 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | C |  | 250 |  |  |  |  | 20.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6EZ8 | GM | T |  |  | 54 |  |  | 980 |  | 00 |  | 6.3 |  |  |  |  | 10 |  |  | M |  |  | D |  | 125 |  |  |  |  | 5.6 |  |  | -------- |  |  |  | RL |  |  |  |  | 17 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 6EZ8 | GM | T |  |  | 54 |  |  | 760 |  | 00 |  | 6.3 |  |  |  |  | 10 |  |  | M |  |  | D |  | 125 |  |  |  |  | 5.6 |  |  | -------- |  |  |  | RL |  |  |  |  | 17 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. |
| 6EZ8 | GM | T |  |  | 54 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 17 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #3. |
| 6FA7 | GM | T |  |  | 45 |  |  | 798 |  | 61 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.4 |  |  | 100 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode #1. |
| 6FA7 | GM | T |  |  | 45 |  |  | 718 |  | 69 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.4 |  |  | 100 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode #2. |
| 6FA7 | EM | T |  |  | 45 |  |  | 930 |  | 61 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. |
| 6FD6 | EM | T |  |  | 34 |  |  | 165 |  | 72 |  | 6.3 |  |  |  |  | 50 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 65 |  |  |  |  | No leakage test. |
| 6FE5 | GM | T |  |  | 72 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 130 |  |  |  |  | 11 |  |  | 130 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FG5 | GM | T |  |  | 43 |  |  | 156 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 250 |  |  |  |  | .2 |  |  | 250 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FG7 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6FG7 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.1 |  |  | -------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6FH5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6FH6 | GM | T |  |  | 72 |  |  | 5A4 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FH8 | GM | T |  |  | 54 |  |  | 697 |  | 00 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | E |  | 100 |  |  |  |  | 8.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 20 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode plate #1. |
| 6FH8 | GM | T |  |  | 54 |  |  | 687 |  | 00 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | E |  | 100 |  |  |  |  | 4.6 |  |  | 100 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode plate #2. |
| 6FH8 | GM | T |  |  | 54 |  |  | 617 |  | 00 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | E |  | 100 |  |  |  |  | 4.6 |  |  | 100 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode plate #3. |
| 6FH8 | EM | T |  |  | 54 |  |  | 230 |  | 00 |  | 6.3 |  |  |  |  | 5 |  |  | R |  |  | A |  | 35 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 19 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6FM8 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6FM8 | EM | T |  |  | 45 |  |  | 060 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6FM8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | -------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6FQ5 | GM | T |  |  | 43 |  |  | 250 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FQ5A | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | B |  | 135 |  |  |  |  | 1.2 |  |  | -------- |  |  |  | RL |  |  |  |  | 31 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FQ7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6FQ7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6FS5 | GM | T |  |  | 34 |  |  | 156 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 245 |  |  |  |  | .2 |  |  | 135 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FV6 | GM | T |  |  | 43 |  |  | 156 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | C |  | 125 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FV8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 6FV8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6FW5 | GM | T |  |  | 27 |  |  | 158 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | J |  |  | C |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 22 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FW8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6FW8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 2.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6FY5 | GM | T |  |  | 34 |  |  | 250 |  | 16 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 135 |  |  |  |  | 1.0 |  |  | -------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6FY8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 125 |  |  |  |  | 10.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 6FY8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 125 |  |  |  |  | 1.5 |  |  | -------- |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6G5 | ER | T |  |  | 16 |  |  | 540 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 230 |  |  |  |  | 0 |  |  | 225 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye open. |
| 6G5 | ER | T |  |  | 16 |  |  | 542 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 230 |  |  |  |  | 0 |  |  | 225 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye closed. |
| 6GC5 | GM | T |  |  | 45 |  |  | 691 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 115 |  |  |  |  | 7.5 |  |  | 115 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6GE8 | GM | T |  |  | 45 |  |  | 891 |  | 76 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 150 |  |  |  |  | 2.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6GE8 | GM | T |  |  | 45 |  |  | 360 |  | 29 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 150 |  |  |  |  | 21.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6GH8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6GH8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6GK5 | GM | T |  |  | 34 |  |  | 250 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | A |  | 135 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6GK6 | GM | T |  |  | 45 |  |  | 278 |  | 13 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.3 |  |  | 250 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6GM5 | GM | T |  |  | 45 |  |  | 691 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6GM6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 175 |  |  |  |  | .8 |  |  | 125 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6GM8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | Q |  |  | B |  | 25 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6GM8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | Q |  |  | B |  | 25 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6GN8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | B |  | 200 |  |  |  |  | 3.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 6GN8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 6GS8 | GM | T |  |  | 54 |  |  | 782 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. |
| 6GS8 | GM | T |  |  | 54 |  |  | 732 |  | 16 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | ------- |  |  | 67.5 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. |
| 6GW6 | GM | T |  |  | 27 |  |  | 5A4 | 80 |  | 6.3 |  |  |  | 50 |  |  | J |  |  | B |  |  | 250 |  |  |  | 22.5 |  |  | 150 |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  |  | 65 |  |  |  | If tube oscillates, turn grid selector to “B.” Connect 1,000, 1/2w carbon resistor between pin 5 and top cap lead B. |  |  |  |
| 6GX6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6GY6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | C |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6GY8 | S | T |  |  | 45 |  |  | 060 |  | 70 |  | OFF |  |  |  |  | OFF |  |  | --------- |  |  | ---------- |  | ----- |  |  |  |  | ------- |  |  | --------- |  |  |  | --------- |  |  |  |  | ------ |  |  |  | ------- |  |  | ----------- |  |  |  |  | Triode #1. Shorts test only. |
| 6GY8 | EM | T |  |  | 45 |  |  | 060 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 125 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Short on V. |
| 6GY8 | GM | T |  |  | 54 |  |  | 390 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 125 |  |  |  |  | 1.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6GY8 | EM | T |  |  | 54 |  |  | 120 |  | 00 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 35 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 12 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #3. |
| 6HB6 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 6.3 |  |  |  |  | B |  |  | G |  |  | A |  | 250 |  |  |  |  | CCW |  |  | 250 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6HF8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6HF8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6HJ8 | GM | T |  |  | 45 |  |  | 263 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | .5 |  |  | 125 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6HJ8 | EM | T |  |  | 45 |  |  | 680 |  | 79 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 84 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. |
| 6HS8 | GM | T |  |  | 45 |  |  | 782 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 97.5 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 84 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #1. |
| 6HS8 | GM | T |  |  | 45 |  |  | 732 |  | 16 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 97.5 |  |  |  |  | 0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 84 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode #2. |
| 6N8 | GM | T |  |  | 45 |  |  | 261 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 180 |  |  |  |  | 2.0 |  |  | 85 |  |  |  | RL |  |  |  |  | 84 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6N8 | EM | T |  |  | 45 |  |  | 271 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6N8 | EM | T |  |  | 45 |  |  | 281 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6R4 | GM | T |  |  | 45 |  |  | 180 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6R8 | GM | T |  |  | 45 |  |  | 890 |  | 72 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 9.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6R8 | EM | T |  |  | 45 |  |  | 010 |  | 78 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6R8 | EM | T |  |  | 45 |  |  | 060 |  | 78 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6R8 | EM | T |  |  | 45 |  |  | 020 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 6S8 | GM | T |  |  | 78 |  |  | B60 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6S8 | EM | T |  |  | 78 |  |  | 030 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6S8 | EM | T |  |  | 78 |  |  | 040 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6S8 | EM | T |  |  | 78 |  |  | 010 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 6X6 | ER | T |  |  | 72 |  |  | 540 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 230 |  |  |  |  | 0 |  |  | 230 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eyes open. |
| 6X6 | ER | T |  |  | 72 |  |  | 543 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | F |  | 230 |  |  |  |  | 0 |  |  | 230 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eyes closed. |
| 7A6 | EM | T |  |  | 18 |  |  | 060 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | D |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 7A6 | EM | T |  |  | 18 |  |  | 030 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | D |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 7AD7 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 3.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7AG7 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7AH7 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7AJ7 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7AU7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 7.0. |
| 7AU7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 7.0. |
| 7EY6 | GM | T |  |  | 72 |  |  | 534 |  | 80 |  | 7.5 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 17.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 7.2. |
| 8AU8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | B |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.4. |
| 8AU8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | C |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8AW8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.7 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.4. |
| 8AW8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | 5 |  |  | H |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8BA8A | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 51 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.4. |
| 8BA8A | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8BH8 | GM | T |  |  | 45 |  |  | 798 |  | 30 |  | 7.5 |  |  |  |  | A |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.4. |
| 8BH8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | 10 |  |  | M |  |  | C |  | 150 |  |  |  |  | 5.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8BN8 | GM | T |  |  | 45 |  |  | 870 |  | 90 |  | 7.5 |  |  |  |  | 5 |  |  | G |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8BN8 | EM | T |  |  | 45 |  |  | 060 |  | 30 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 8.4. |
| 8BN8 | EM | T |  |  | 45 |  |  | 010 |  | 20 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 8.4. |
| 8BQ5 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.2 |  |  | 250 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 8.0. |
| 8CG7 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 8.4. |
| 8CG7 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 8.4. |
| 8CM7 | GM | T |  |  | 45 |  |  | 760 |  | 30 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 200 |  |  |  |  | 7.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 8.4. |
| 8CM7 | GM | T |  |  | 45 |  |  | 810 |  | 90 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 36 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 8.4. |
| 8CN7 | GM | T |  |  | 45 |  |  | 780 |  | 60 |  | 7.5 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.4. |
| 8CN7 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 8.4. |
| 8CN7 | EM | T |  |  | 45 |  |  | 010 |  | 30 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 8.4. |
| 8CS7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 7.5 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 8.4. |
| 8CS7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 7.5 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 10.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 8.4. |
| 8CX8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | A |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.0. |
| 8CX8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | C |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.0. |
| 8CY7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 7.5 |  |  |  |  | 5 |  |  | J |  |  | D |  | 235 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 7.9. |
| 8CY7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 7.5 |  |  |  |  | E |  |  | N |  |  | B |  | 90 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 7.9. |
| 8EB8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | B |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 8.0. |
| 8EB8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 7.5 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 8.0. |
| 8EM5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 7.5 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 18.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 8.0. |
| 8ET7 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | 5 |  |  | L |  |  | B |  | 200 |  |  |  |  | 2.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 29 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 8ET7 | EM | T |  |  | 45 |  |  | 030 |  | 10 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 8ET7 | EM | T |  |  | 45 |  |  | 020 |  | 10 |  | 7.5 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 8FQ7 | GM | T |  |  | 45 |  |  | 700 |  | 80 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 8.4V. |
| 8FQ7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 8.4V. |
| 8GN8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 7.5 |  |  |  |  | 5 |  |  | L |  |  | B |  | 200 |  |  |  |  | 3.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. Adj fil to 8.0. |
| 8GN8 | GM | T |  |  | 45 |  |  | 230 |  | 30 |  | 7.5 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. Adj fil to 8.0. |
| 8SN7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 8.4. |
| 8SN7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 7.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 8.4. |
| 9AU7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 10 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 9.4. |
| 9AU7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 10 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 9.4. |
| 9BR7 | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 10.0 |  |  |  |  | E |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 9.4. |
| 9BR7 | EM | T |  |  | 54 |  |  | 070 |  | 80 |  | 10.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. Adj fil to 9.4. |
| 9BR7 | EM | T |  |  | 54 |  |  | 060 |  | 80 |  | 10.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. Adj fil to 9.4. |
| 9CL8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 10 |  |  |  |  | 5 |  |  | G |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. Adj fil to 9.5. |
| 9CL8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 10 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 9.5. |
| 9DZ8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 10 |  |  |  |  | D |  |  | K |  |  | B |  | 145 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 9.0. |
| 9DZ8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 10 |  |  |  |  | 5 |  |  | K |  |  | D |  | 120 |  |  |  |  | 1.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 9.0. |
| 9U8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 10 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 9.45. |
| 9U8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 10 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 9.45. |
| 9X8 | GM | T |  |  | 45 |  |  | 798 |  | 61 |  | 10 |  |  |  |  | D |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 9.5. |
| 9X8 | GM | T |  |  | 45 |  |  | 230 |  | 60 |  | 10 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 9.5. |
| 10BQ5 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 10.0 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.2 |  |  | 250 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 10.6. |
| 10C8 | GM | T |  |  | 45 |  |  | 867 |  | 90 |  | 10 |  |  |  |  | B |  |  | G |  |  | B |  | 135 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 10.5. |
| 10C8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 10 |  |  |  |  | E |  |  | G |  |  | C |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 10.5. |
| 10DA7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 10 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 10DA7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 10 |  |  |  |  | 10 |  |  | K |  |  | B |  | 150 |  |  |  |  | 17.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 10DE7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 10 |  |  |  |  | 50 |  |  | J |  |  | C |  | 225 |  |  |  |  | 12.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 10DE7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 10 |  |  |  |  | 50 |  |  | K |  |  | B |  | 150 |  |  |  |  | 26.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 10DR7 | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 10.0 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 10DR7 | GM | T |  |  | 54 |  |  | 210 |  | 90 |  | 10.0 |  |  |  |  | 50 |  |  | M |  |  | C |  | 150 |  |  |  |  | 17.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 10EG7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 10.0 |  |  |  |  | 50 |  |  | J |  |  | C |  | 225 |  |  |  |  | 12.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 10EG7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 10.0 |  |  |  |  | 50 |  |  | M |  |  | B |  | 150 |  |  |  |  | 17.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. |
| 10EM7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 10.0 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 9.7. |
| 10EM7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 10.0 |  |  |  |  | 50 |  |  | L |  |  | B |  | 150 |  |  |  |  | 20.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 9.7. |
| 10IF8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 10.0 |  |  |  |  | B |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 10.5. |
| 10IF8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 10.0 |  |  |  |  | 5 |  |  | J |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 10.5. |
| 11C5 | GM | T |  |  | 34 |  |  | 576 |  | 10 |  | 10 |  |  |  |  | 50 |  |  | K |  |  | B |  | 135 |  |  |  |  | 8.2 |  |  | 120 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 11.6. |
| 11CY7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 10 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 11.6. |
| 11CY7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 10 |  |  |  |  | E |  |  | N |  |  | B |  | 90 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 11.6. |
| 12AB5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12AC6 | EM | T |  |  | 34 |  |  | 256 |  | 71 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 81 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12AD6 | EM | T |  |  | 34 |  |  | 756 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12AD7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 12AD7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12AE6 | EM | T |  |  | 34 |  |  | 070 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. |
| 12EA6 | EM | T |  |  | 34 |  |  | 060 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #1. |
| 12AE6 | EM | T |  |  | 34 |  |  | 050 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. |
| 12AE7 | EM | T |  |  | 45 |  |  | 060 |  | 87 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #1. |
| 12AE7 | EM | T |  |  | 45 |  |  | 010 |  | 32 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #2. |
| 12AF6 | EM | T |  |  | 34 |  |  | 256 |  | 71 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12AG6 | EM | T |  |  | 34 |  |  | 756 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12AJ6 | EM | T |  |  | 34 |  |  | 070 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. |
| 12AJ6 | EM | T |  |  | 34 |  |  | 060 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #1. |
| 12AJ6 | EM | T |  |  | 34 |  |  | 050 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. |
| 12AJ7 | GM | T |  |  | 45 |  |  | 261 |  | 37 |  | 12.6 |  |  |  |  | 5 |  |  | L |  |  | C |  | 200 |  |  |  |  | 1.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  | Heptode sect. |
| 12AJ7 | GM | T |  |  | 45 |  |  | 980 |  | 36 |  | 12.6 |  |  |  |  | 5 |  |  | L |  |  | C |  | 150 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12AL8 | EM | T |  |  | 45 |  |  | 312 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12AL8 | EM | T |  |  | 45 |  |  | 860 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Triode sect. |
| 12AQ5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12AS5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | L |  |  | C |  | 150 |  |  |  |  | 8.3 |  |  | 110 |  |  |  | RL |  |  |  |  | 36 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12AU8 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 12.6 |  |  |  |  | B |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12AU8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 12.6 |  |  |  |  | D |  |  | M |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12AV5 | GM | T |  |  | 27 |  |  | 158 |  | 30 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BD6 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BK5 | GM | T |  |  | 45 |  |  | 318 |  | 60 |  | 12.6 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 5.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BL6 | EM | T |  |  | 34 |  |  | 256 |  | 71 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P2 |  |  | 50 |  |  |  |  |  |
| 12BN6 | GM | T |  |  | 34 |  |  | 275 |  | 16 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | D |  | 75 |  |  |  |  | 2.0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BQ6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BR7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | D |  |  | G |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12BR7 | EM | T |  |  | 45 |  |  | 070 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12BR7 | EM | T |  |  | 45 |  |  | 060 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12BV7 | GM | T |  |  | 45 |  |  | 278 |  | 13 |  | 12.6 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BZ6 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 12.6 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12BZ7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 12BZ7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12C5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CA5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | B |  | 125 |  |  |  |  | 4.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CM6 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CN5 | EM | T |  |  | 34 |  |  | 670 |  | 12 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 32 |  |  |  | P2 |  |  | 50 |  |  |  |  |  |
| 12CR6 | GM | T |  |  | 34 |  |  | 756 |  | 10 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 200 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12CR6 | EM | T |  |  | 34 |  |  | 020 |  | 10 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 12CS6 | GM | T |  |  | 34 |  |  | 165 |  | 27 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | C |  | 80 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CT8 | GM | T |  |  | 45 |  |  | 867 |  | 90 |  | 12.6 |  |  |  |  | B |  |  | L |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 130 |  |  |  | RL |  |  |  |  | 69 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12CT8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | C |  |  | L |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12CU5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 50 |  |  | K |  |  | B |  | 120 |  |  |  |  | 8.0 |  |  | 110 |  |  |  | RL |  |  |  |  | 64 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CU6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12CX6 | EM | T |  |  | 34 |  |  | 165 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12DB5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 10 |  |  | N |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12DE8 | EM | T |  |  | 45 |  |  | 168 |  | 97 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 22 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12DE8 | EM | T |  |  | 45 |  |  | 030 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 12DF7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 12DF7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12DK7 | EM | T |  |  | 45 |  |  | 173 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 18 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12DK7 | EM | T |  |  | 45 |  |  | 060 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12DK7 | EM | T |  |  | 45 |  |  | 090 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12DL8 | EM | T |  |  | 45 |  |  | 763 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 17 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12DL8 | EM | T |  |  | 45 |  |  | 090 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 46 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12DL8 | EM | T |  |  | 45 |  |  | 010 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 46 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12DM5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12DQ6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12DQ7 | GM | T |  |  | 45 |  |  | 278 |  | 19 |  | 12.6 |  |  |  |  | A |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12DT5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 16.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12DT7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 12DT7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12DT8 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 12DT8 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12DU7 | EM | T |  |  | 54 |  |  | 163 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 59 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. Make no gas test. |
| 12DU7 | EM | T |  |  | 54 |  |  | 090 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 12DU7 | EM | T |  |  | 54 |  |  | 070 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 12DV7 | EM | T |  |  | 54 |  |  | 060 |  | 87 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 48 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. Make no gas test. |
| 12DV7 | EM | T |  |  | 54 |  |  | 030 |  | 10 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 12DV7 | EM | T |  |  | 54 |  |  | 020 |  | 10 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 12DV8 | EM | T |  |  | 45 |  |  | 763 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12DV8 | EM | T |  |  | 45 |  |  | 090 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12DV8 | EM | T |  |  | 45 |  |  | 010 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12DW5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 50 |  |  | H |  |  | C |  | 210 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 60 |  |  |  |  |  |
| 12DW7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 255 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 67 |  |  |  |  | Triode #1. |
| 12DW7 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 12DW8 | EM | T |  |  | 54 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #1. |
| 12DW8 | EM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 17 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #2. |
| 12DY8 | EM | T |  |  | 45 |  |  | 163 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12DY8 | EM | T |  |  | 45 |  |  | 980 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Triode sect. |
| 12DZ6 | EM | T |  |  | 45 |  |  | 156 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 96 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12DZ8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 12.6 |  |  |  |  | D |  |  | K |  |  | B |  | 145 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12DZ8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | K |  |  | D |  | 120 |  |  |  |  | 1.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12EA6 | EM | T |  |  | 43 |  |  | 156 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12EC8 | EM | T |  |  | 45 |  |  | 967 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12EC8 | EM | T |  |  | 45 |  |  | 102 |  | 60 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Triode sect. |
| 12ED5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | B |  | 125 |  |  |  |  | 4.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12EG6 | EM | T |  |  | 34 |  |  | 756 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 32 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12EH5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | B |  | 115 |  |  |  |  | 4.8 |  |  | 110 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12EK6 | EM | T |  |  | 43 |  |  | 156 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12EL6 | EM | T |  |  | 43 |  |  | 020 |  | 71 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P2 |  |  | 40 |  |  |  |  | Triode sect. |
| 12EL6 | EM | T |  |  | 43 |  |  | 060 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #1. |
| 12EL6 | EM | T |  |  | 43 |  |  | 050 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. |
| 12EM6 | EM | T |  |  | 45 |  |  | 163 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 18 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12EM6 | EM | T |  |  | 45 |  |  | 090 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | F |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode sect. |
| 12EN6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 12.6 |  |  |  |  | 10 |  |  | M |  |  | B |  | 200 |  |  |  |  | 9.0 |  |  | 110 |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12EQ7 | GM | T |  |  | 45 |  |  | 276 |  | 31 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 0.6 |  |  | 100 |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12EQ7 | EM | T |  |  | 45 |  |  | 080 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Diode sect. |
| 12EZ6 | EM | T |  |  | 43 |  |  | 156 |  | 72 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0.7 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  |  |
| 12F8 | EM | T |  |  | 45 |  |  | 239 |  | 78 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12F8 | EM | T |  |  | 45 |  |  | 060 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 61 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #1. |
| 12F8 | EM | T |  |  | 45 |  |  | 010 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 61 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. |
| 12FK6 | EM | T |  |  | 43 |  |  | 170 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | C |  | 20 |  | AC |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 36 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. |
| 12FK6 | EM | T |  |  | 43 |  |  | 060 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 12FK6 | EM | T |  |  | 43 |  |  | 050 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 49 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 12FM6 | EM | T |  |  | 34 |  |  | 070 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 67 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. Make no gas test. |
| 12FM6 | EM | T |  |  | 34 |  |  | 060 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 12FM6 | EM | T |  |  | 34 |  |  | 050 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 12FQ8 | GM | T |  |  | 45 |  |  | 780 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. Plate #1. |
| 12FQ8 | GM | T |  |  | 45 |  |  | 760 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. Plate #2. |
| 12FQ8 | GM | T |  |  | 45 |  |  | 230 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. Plate #1. |
| 12FQ8 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. Plate #2. |
| 12FR8 | EM | T |  |  | 45 |  |  | 367 |  | 00 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | D |  | 20 |  | AC |  |  | 3.1 |  |  | --------- |  |  |  | RL |  |  |  |  | 10 |  |  |  | P2 |  |  | 65 |  |  |  |  | Pentode sect. |
| 12FR8 | EM | T |  |  | 45 |  |  | 190 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | D |  | 20 |  | AC |  |  | .5 |  |  | --------- |  |  |  | RL |  |  |  |  | 27 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. |
| 12FR8 | EM | T |  |  | 45 |  |  | 080 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | D |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode sect. |
| 12FT6 | EM | T |  |  | 43 |  |  | 170 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 64 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. Make no gas test. |
| 12FT6 | EM | T |  |  | 43 |  |  | 060 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12FT6 | EM | T |  |  | 43 |  |  | 050 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12FX8 | EM | T |  |  | 45 |  |  | 231 |  | 79 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 20 |  |  |  | P2 |  |  | 65 |  |  |  |  | Heptode sect. |
| 12FX8 | EM | T |  |  | 45 |  |  | 080 |  | 06 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 18 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode sect. |
| 12G4 | GM | T |  |  | 34 |  |  | 610 |  | 70 |  | 12.6 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12G8 | EM | T |  |  | 45 |  |  | 060 |  | 78 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #1. |
| 12G8 | EM | T |  |  | 45 |  |  | 010 |  | 23 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #2. |
| 12GA6 | EM | T |  |  | 34 |  |  | 756 |  | 21 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P2 |  |  | 65 |  |  |  |  |  |
| 12GC6 | GM | T |  |  | 72 |  |  | 5A8 |  | 30 |  | 12.6 |  |  |  |  | 500 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12GW6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.6 |  |  | 150 |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 65 |  |  |  |  | If tube oscillates, turn grid selector to “B.” Connect a 1,000, 1/2w carbon resistor between pin 5 and top cap lead B. |
| 12J8 | EM | T |  |  | 45 |  |  | 163 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Tetrode sect. |
| 12J8 | EM | T |  |  | 45 |  |  | 090 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12J8 | EM | T |  |  | 45 |  |  | 080 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12K5 | EM | T |  |  | 34 |  |  | 570 |  | 12 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 91 |  |  |  | P2 |  |  | 65 |  |  |  |  |  |
| 12L6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 12.6 |  |  |  |  | C |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12R5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 12.6 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.5 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12S8 | GM | T |  |  | 78 |  |  | B60 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 12S8 | EM | T |  |  | 78 |  |  | 030 |  | 50 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 51 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 12S8 | EM | T |  |  | 78 |  |  | 040 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 51 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 12S8 | EM | T |  |  | 78 |  |  | 010 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 51 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #3. |
| 12U7 | EM | T |  |  | 45 |  |  | 060 |  | 87 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #1. |
| 12U7 | EM | T |  |  | 45 |  |  | 010 |  | 32 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 43 |  |  |  | P2 |  |  | 65 |  |  |  |  | Triode #2. |
| 12V6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 12W6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 12.6 |  |  |  |  | C |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 13DE7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | J |  |  | C |  | 225 |  |  |  |  | 12.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 13.0. |
| 13DE7 | GM | T |  |  | 45 |  |  | 210 |  | 90 |  | 12.6 |  |  |  |  | 50 |  |  | K |  |  | B |  | 150 |  |  |  |  | 26.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 13.0. |
| 13DR7 | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 13.0. |
| 13DR7 | GM | T |  |  | 54 |  |  | 210 |  | 90 |  | 12.6 |  |  |  |  | 50 |  |  | M |  |  | C |  | 150 |  |  |  |  | 17.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 13.0. |
| 13EM7 | GM | T |  |  | 87 |  |  | 450 |  | 60 |  | 14.0 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 13.0. |
| 13EM7 | GM | T |  |  | 87 |  |  | 120 |  | 30 |  | 14.0 |  |  |  |  | 50 |  |  | K |  |  | C |  | 150 |  |  |  |  | 20 |  |  | --------- |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 13.0. |
| 13FR7 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 13.0. |
| 13FR7 | GM | T |  |  | 45 |  |  | 310 |  | 90 |  | 12.6 |  |  |  |  | 50 |  |  | K |  |  | C |  | 150 |  |  |  |  | 20.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 13.0. |
| 14GT8 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 14.0 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 97 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 14GT8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 14GT8 | EM | T |  |  | 45 |  |  | 060 |  | 10 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 14JG8 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 14.0 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 14JG8 | EM | T |  |  | 45 |  |  | 060 |  | 10 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 14JG8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 14V7 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 12.6 |  |  |  |  | C |  |  | H |  |  | C |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 14X7 | GM | T |  |  | 18 |  |  | 320 |  | 40 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 97 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 14X7 | EM | T |  |  | 18 |  |  | 050 |  | 40 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 14X7 | EM | T |  |  | 18 |  |  | 060 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 15A6 | GM | T |  |  | 45 |  |  | 271 |  | 36 |  | 14.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 180 |  |  |  |  | 2.9 |  |  | 180 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 15.0. |
| 15EA7 | GM | T |  |  | 78 |  |  | 450 |  | 60 |  | 14.0 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 14.8. |
| 15EA7 | GM | T |  |  | 78 |  |  | 120 |  | 30 |  | 14.0 |  |  |  |  | 50 |  |  | L |  |  | B |  | 175 |  |  |  |  | 25.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 14.8. |
| 15EW6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 14.0 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 15.0. |
| 16A5 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 14.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 170 |  |  |  |  | 10.4 |  |  | 170 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.5. |
| 16A5 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 14.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 170 |  |  |  |  | 10.5 |  |  | 165 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.5. Disregard shorts test after GM test. Allow 10 second intervals for zeroing quality meter and 10 second intervals between attempts to zero quality meter. Allow 5 seconds to obtain GM reading. |
| 16A8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 14.0 |  |  |  |  | 50 |  |  | G |  |  | B |  | 200 |  |  |  |  | 16.0 |  |  | 200 |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 16.0. |
| 16A8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 14.0 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 16.0. |
| 17AV5 | GM | T |  |  | 27 |  |  | 158 |  | 30 |  | 20 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17BQ6 | GM | T |  |  | 21 |  |  | 5A4 |  | 80 |  | 20 |  |  |  |  | 50 |  |  | J |  |  | C |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17C5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 20 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17CA5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 20 |  |  |  |  | 10 |  |  | K |  |  | B |  | 125 |  |  |  |  | 4.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17DQ6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 20 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17EW8 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 200 |  |  |  |  | 2.1 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. Adj fil to 17.5. |
| 17EW8 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 200 |  |  |  |  | 2.1 |  |  | --------- |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. Adj fil to 17.5. |
| 17GW6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 20.0 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. If tube oscillates, turn grid selector to “B.” Connect a 1,000, 1/2w carbon resistor between pin 5 and top cap lead B. |
| 17L6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 20.0 |  |  |  |  | C |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 17R5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 20.0 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.5 |  |  | 100 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 16.8. |
| 18A5 | GM | T |  |  | 27 |  |  | 158 |  | 30 |  | 20.0 |  |  |  |  | 50 |  |  | J |  |  | C |  | 200 |  |  |  |  | 17.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 18.5. |
| 18DZ8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 20.0 |  |  |  |  | D |  |  | K |  |  | B |  | 145 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 18.0. |
| 18DZ8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | D |  | 120 |  |  |  |  | 1.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.0. |
| 18FW6 | GM | T |  |  | 43 |  |  | 156 |  | 70 |  | 20.0 |  |  |  |  | B |  |  | N |  |  | C |  | 110 |  |  |  |  | CCW |  |  | 110 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 18.0. |
| 18FX6 | GM | T |  |  | 43 |  |  | 156 |  | 27 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 110 |  |  |  |  | 3.7 |  |  | 110 |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  | Ampl sect. Adj fil to 18.0. |
| 18FX6 | GM | T |  |  | 43 |  |  | 160 |  | 27 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | B |  | 100 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | OSC sect. Adj fil to 18.0. |
| 18FY6 | GM | T |  |  | 43 |  |  | 170 |  | 20 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.0. |
| 18FY6 | EM | T |  |  | 43 |  |  | 060 |  | 20 |  | 20.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 48 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. Adj fil to 18.0. |
| 18FY6 | EM | T |  |  | 43 |  |  | 050 |  | 20 |  | 20.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 48 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. Adj fil to 18.0. |
| 18HB8 | GM | T |  |  | 45 |  |  | 976 |  | 80 |  | 20.0 |  |  |  |  | 10 |  |  | K |  |  | C |  | 115 |  |  |  |  | 6.3 |  |  | 115 |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 18.0. |
| 18HB8 | GM | T |  |  | 45 |  |  | 130 |  | 20 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | C |  | 115 |  |  |  |  | 1.0 |  |  | 115 |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.0. |
| 19AQ5 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 20 |  |  |  |  | 10 |  |  | K |  |  | C |  | 180 |  |  |  |  | 8.5 |  |  | 180 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 18.9. |
| 19BG6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 20 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 22.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 51 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 18.9. |
| 19C8 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 20 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.9. |
| 19C8 | EM | T |  |  | 45 |  |  | 060 |  | 70 |  | 20 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P4 |  |  | 65 |  |  |  |  | Diode #1. Adj fil to 18.9. |
| 19C8 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 20 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. Adj fil to 18.9. |
| 19C8 | EM | T |  |  | 45 |  |  | 010 |  | 70 |  | 20 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #3. Adj fil to 18.9. |
| 19CL8A | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode sect. Adj fil to 18.9. |
| 19CL8A | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 20.0 |  |  |  |  | A |  |  | K |  |  | B |  | 125 |  |  |  |  | .7 |  |  | --------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.9. |
| 19EA8 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 150 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 18.9. |
| 19EA8 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 20.0 |  |  |  |  | A |  |  | K |  |  | B |  | 150 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.9. |
| 19EZ8 | GM | T |  |  | 45 |  |  | 980 |  | 40 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | C |  | 125 |  |  |  |  | .3 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 18.9. |
| 19EZ8 | GM | T |  |  | 45 |  |  | 760 |  | 40 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | C |  | 125 |  |  |  |  | .3 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 18.9. |
| 19EZ8 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | C |  | 125 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #3. Adj fil to 18.9. |
| 19HV8 | GM | T |  |  | 45 |  |  | 967 |  | 80 |  | 20.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. Adj fil to 18.9. |
| 19HV8 | GM | T |  |  | 45 |  |  | 120 |  | 30 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. Adj fil to 18.9. |
| 19J6 | GM | T |  |  | 34 |  |  | 610 |  | 75 |  | 20 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 18.9. |
| 19J6 | GM | T |  |  | 34 |  |  | 520 |  | 76 |  | 20 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 18.9. |
| 20 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 3.0 |  |  |  |  | 50 |  |  | M |  |  | E |  | 135 |  |  |  |  | 22.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.3. |
| 20EQ7 | GM | T |  |  | 45 |  |  | 276 |  | 31 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | .6 |  |  | 100 |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 20EQ7 | EM | T |  |  | 45 |  |  | 080 |  | 30 |  | 20.0 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P2 |  |  | 25 |  |  |  |  | Triode sect. |
| 20EZ7 | GM | T |  |  | 12 |  |  | 870 |  | 90 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | B |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 20EZ7 | GM | T |  |  | 12 |  |  | 560 |  | 40 |  | 20.0 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 21A6 | GM | T |  |  | 45 |  |  | 2A7 |  | 31 |  | 20 |  |  |  |  | 50 |  |  | K |  |  | B |  | 180 |  |  |  |  | 23.0 |  |  | 180 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 21.5. |
| 21EX6 | GM | T |  |  | 72 |  |  | 5A8 |  | 30 |  | 20.0 |  |  |  |  | 50 |  |  | G |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  | Adj fil to 21.5. Tubes showing shorts: Retest using 72 5A1 30. |
| 25C5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 26 |  |  |  |  | 10 |  |  | N |  |  | B |  | 100 |  |  |  |  | 6.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25CA5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 26 |  |  |  |  | 10 |  |  | K |  |  | B |  | 125 |  |  |  |  | 4.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25CD6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 26 |  |  |  |  | 50 |  |  | K |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25CU6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 26 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25DN6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 26 |  |  |  |  | 50 |  |  | K |  |  | B |  | 125 |  |  |  |  | 18.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25DQ6 | GM | T |  |  | 27 |  |  | 5A4 |  | 80 |  | 26 |  |  |  |  | 50 |  |  | J |  |  | B |  | 250 |  |  |  |  | 22.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25DT5 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 26.0 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 16.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25EC6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 26.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 135 |  |  |  |  | 22.5 |  |  | 135 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25EH5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 26 |  |  |  |  | 10 |  |  | K |  |  | B |  | 115 |  |  |  |  | 4.8 |  |  | 110 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25F5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 26 |  |  |  |  | 10 |  |  | N |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 25W6 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 26 |  |  |  |  | C |  |  | J |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 27S | GM | T |  |  | 15 |  |  | 320 |  | 40 |  | 2.5 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 21.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 32ET5 | GM | T |  |  | 43 |  |  | 276 |  | 10 |  | 35.0 |  |  |  |  | 10 |  |  | K |  |  | B |  | 115 |  |  |  |  | 7.0 |  |  | 110 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 32.0. |
| 34GD5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 35.0 |  |  |  |  | 10 |  |  | N |  |  | C |  | 110 |  |  |  |  | 6.6 |  |  | 110 |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 34.0. |
| 35 | GM | T |  |  | 15 |  |  | B23 |  | 40 |  | 2.5 |  |  |  |  | 5 |  |  | M |  |  | D |  | 180 |  |  |  |  | 3.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 35CD6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 35 |  |  |  |  | 50 |  |  | K |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 35DZ8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 35 |  |  |  |  | D |  |  | K |  |  | B |  | 145 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 35DZ8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 35 |  |  |  |  | 5 |  |  | K |  |  | D |  | 120 |  |  |  |  | 1.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 35EH5 | GM | T |  |  | 43 |  |  | 276 |  | 10 |  | 35.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 110 |  |  |  |  | 2.0 |  |  | 115 |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 35GL6 | GM | T |  |  | 34 |  |  | 275 |  | 10 |  | 35.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 35HB8 | GM | T |  |  | 45 |  |  | 976 |  | 80 |  | 35.0 |  |  |  |  | 10 |  |  | K |  |  | C |  | 115 |  |  |  |  | 6.8 |  |  | 115 |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 35HB8 | GM | T |  |  | 45 |  |  | 130 |  | 20 |  | 35.0 |  |  |  |  | 5 |  |  | K |  |  | C |  | 115 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 35S | GM | T |  |  | 15 |  |  | A23 |  | 40 |  | 2.5 |  |  |  |  | 5 |  |  | M |  |  | D |  | 180 |  |  |  |  | 3.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 40FR5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 35.0 |  |  |  |  | 10 |  |  | N |  |  | B |  | 110 |  |  |  |  | 7.5 |  |  | 110 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 40.0 |
| 50BK5 | GM | T |  |  | 45 |  |  | 318 |  | 60 |  | 50 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 5.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 50CD6 | GM | T |  |  | 27 |  |  | 5A8 |  | 30 |  | 50 |  |  |  |  | 50 |  |  | K |  |  | B |  | 175 |  |  |  |  | 30.0 |  |  | 175 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 50EH5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 50 |  |  |  |  | 10 |  |  | K |  |  | B |  | 115 |  |  |  |  | 4.8 |  |  | 110 |  |  |  | RL |  |  |  |  | 49 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 50FA5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 50.0 |  |  |  |  | 10 |  |  | N |  |  | C |  | 105 |  |  |  |  | 5.8 |  |  | 105 |  |  |  | RL |  |  |  |  | 26 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 50FE5 | GM | T |  |  | 72 |  |  | 534 |  | 80 |  | 50.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 130 |  |  |  |  | 11.0 |  |  | 130 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 50FK5 | GM | T |  |  | 34 |  |  | 276 |  | 10 |  | 50.0 |  |  |  |  | B |  |  | N |  |  | B |  | 110 |  |  |  |  | 0 |  |  | 115 |  |  |  | RL |  |  |  |  | 32 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 60FX5 | GM | T |  |  | 43 |  |  | 276 |  | 10 |  | 70.0 |  |  |  |  | 5 |  |  | K |  |  | B |  | 115 |  |  |  |  | 3.2 |  |  | 115 |  |  |  | RL |  |  |  |  | 22 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 60.0 |
| 50FY8 | GM | T |  |  | 45 |  |  | 367 |  | 20 |  | 50.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 125 |  |  |  |  | 10 |  |  | 125 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 50FY8 | GM | T |  |  | 45 |  |  | 190 |  | 80 |  | 50.0 |  |  |  |  | 5 |  |  | M |  |  | D |  | 125 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| HD51 | VR | VR |  |  | 00 |  |  | 001 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 30 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 126 |  |  |  |  | Max. Max. diff -4.5. |
| HD51 | VR | VR |  |  | 00 |  |  | 001 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | H |  |  | F |  | 5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 105 |  |  |  |  | Min. Min. diff -4.5. |
| 51/51S | GM | T |  |  | 15 |  |  | B23 |  | 40 |  | 2.5 |  |  |  |  | 5 |  |  | M |  |  | D |  | 180 |  |  |  |  | 3.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| KT66 | GM | T |  |  | 27 |  |  | 534 |  | 81 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| KT88 | GM | T |  |  | 27 |  |  | 534 |  | 81 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 25.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| V99 | GM | T |  |  | 24 |  |  | 130 |  | 00 |  | 3.0 |  |  |  |  | 10 |  |  | N |  |  | E |  | 90 |  |  |  |  | 4.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.3. |
| X99 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 3.0 |  |  |  |  | 10 |  |  | N |  |  | E |  | 90 |  |  |  |  | 4.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 3.3. |
| 112A | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 5.0 |  |  |  |  | 50 |  |  | G |  |  | C |  | 180 |  |  |  |  | 13.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 93 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| HY114 | GM | T |  |  | 27 |  |  | BA0 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | G |  |  | D |  | 180 |  |  |  |  | 4.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Right cap=P. Left cap=G. |
| 117M7 | GM | T |  |  | 27 |  |  | 435 |  | 80 |  | 117 |  |  |  |  | 10 |  |  | N |  |  | C |  | 105 |  |  |  |  | 5.2 |  |  | 105 |  |  |  | RL |  |  |  |  | 39 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 117M7 | EM | T |  |  | 27 |  |  | 060 |  | 10 |  | 117 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P2 |  |  | 50 |  |  |  |  | Rect. sect. |
| 117Z6GT | EM | T |  |  | 72 |  |  | 030 |  | 40 |  | 117 |  |  |  |  | 5 |  |  | R |  |  | A |  | 35 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P2 |  |  | 80 |  |  |  |  | Diode #2. |
| 117Z6GT | EM | T |  |  | 72 |  |  | 050 |  | 80 |  | 117 |  |  |  |  | 5 |  |  | R |  |  | A |  | 35 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P2 |  |  | 80 |  |  |  |  | Diode #1. |
| 150B2 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 15 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 116 |  |  |  |  | Max. Max. diff -4. |
| 150B2 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | K |  |  | F |  | 5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 107 |  |  |  |  | Min. Min. diff -4. |
| X-155 | GM | T |  |  | 45 |  |  | 760 |  | 89 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| X-155 | GM | T |  |  | 45 |  |  | 210 |  | 39 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| GL502 | TH | VR |  |  | 27 |  |  | 503 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | F |  | ----- |  |  |  |  | * 5 |  |  | 150 |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | Bias |  |  |  |  | Volts, 4 min, 2.8 max |
| CK502AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 1.88 |  |  | 40 |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK503AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.6 |  |  | 45 |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK506AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 10 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 5.1 |  |  | 45 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK518AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.6 |  |  | 40 |  |  |  | RL |  |  |  |  | 51 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK535AX | EM | T |  |  | 53 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 28 |  |  |  | P2,P3 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK541AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.0 |  |  | 40 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| CK546AX | EM | T |  |  | 53 |  |  | 412 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | Q |  |  | C |  | 50 |  |  |  |  | 1.7 |  |  | 50 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P5 |  |  | 63 |  |  |  |  | Adj fil to 1.25. |
| CK547AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 45 |  |  |  |  | 2.0 |  |  | 40 |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| CK548AX | GM | T |  |  | 35 |  |  | 412 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | E |  | 40 |  |  |  |  | 3.0 |  |  | 35 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| CK573AX | GM | T |  |  | 24 |  |  | 310 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | N |  |  | D |  | 90 |  |  |  |  | 4.6 |  |  | --------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.25. |
| 879 | EM | T |  |  | 14 |  |  | 0A0 |  | 00 |  | 2.5 |  |  |  |  | 5 |  |  | G |  |  | A |  | 130 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 24 |  |  |  | P2 |  |  | 50 |  |  |  |  |  |
| 829B | GM | T |  |  | 71 |  |  | 6B3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | B |  | 225 |  |  |  |  | 12 |  |  | 200 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 64 |  |  |  |  | Pentode #1 rt pin. |
| 829B | GM | T |  |  | 71 |  |  | 2A3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | B |  | 225 |  |  |  |  | 12 |  |  | 200 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 64 |  |  |  |  | Pentode #2 left pin. |
| 1007 | EM | T |  |  | 78 |  |  | 030 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | G |  |  | A |  | 130 |  |  |  |  | 0 |  |  | --------- |  |  |  | --------- |  |  |  |  | 10 |  |  |  | P2 |  |  | 90 |  |  |  |  | Diode #1. Adj fil to 1.0. |
| 1007 | EM | T |  |  | 78 |  |  | 050 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | G |  |  | A |  | 130 |  |  |  |  | 0 |  |  | --------- |  |  |  | --------- |  |  |  |  | 10 |  |  |  | P2 |  |  | 90 |  |  |  |  | Diode #2. Adj fil to 1.0. |
| E1148 | GM | T |  |  | 27 |  |  | BA0 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 6.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Near cap=G. Far cap=P. |
| 1201 | GM | T |  |  | 28 |  |  | 130 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 180 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1203 | EM | T |  |  | 18 |  |  | 040 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 29 |  |  |  | P2 |  |  | 50 |  |  |  |  |  |
| 1204 | GM | T |  |  | 27 |  |  | 531 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1206 | GM | T |  |  | 18 |  |  | 573 |  | 62 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 225 |  |  |  |  | 2.5 |  |  | 100 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode #1. |
| 1206 | GM | T |  |  | 18 |  |  | 423 |  | 67 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 225 |  |  |  |  | 2.5 |  |  | 100 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  | Tetrode #2. |
| 1229 | GM | T |  |  | 14 |  |  | B23 |  | 00 |  | 2.0 |  |  |  |  | 10 |  |  | M |  |  | E |  | 135 |  |  |  |  | 4.0 |  |  | 67.5 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1230 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 2.0 |  |  |  |  | 50 |  |  | G |  |  | D |  | 180 |  |  |  |  | 14.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1232 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 225 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1273 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1280 | GM | T |  |  | 18 |  |  | 623 |  | 74 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1282 | GM | T |  |  | 18 |  |  | 623 |  | 45 |  | 6.3 |  |  |  |  | C |  |  | H |  |  | C |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1291 | GM | T |  |  | 84 |  |  | 670 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | M |  |  | D |  | 135 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 1.4. |
| 1291 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | M |  |  | D |  | 135 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 1.4. |
| 1299 | GM | T |  |  | 18 |  |  | 623 |  | 00 |  | 2.5 |  |  |  |  | 10 |  |  | N |  |  | C |  | 90 |  |  |  |  | 5.9 |  |  | 90 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to .8. |
| 1602 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 7.5 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 23.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1610 | GM | T |  |  | 15 |  |  | 324 |  | 00 |  | 2.5 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 16.3 |  |  | 250 |  |  |  | RL |  |  |  |  | 79 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1611 | GM | T |  |  | 27 |  |  | 534 |  | 81 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 16.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1642 | GM | T |  |  | 17 |  |  | 450 |  | 60 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 16.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 1642 | GM | T |  |  | 17 |  |  | B30 |  | 20 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 16.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 1650 | GM | T |  |  | 16 |  |  | 430 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 7.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1655 | GM | T |  |  | 78 |  |  | 450 |  | 62 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 1655 | GM | T |  |  | 78 |  |  | 320 |  | 65 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 72 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 1657 | TH | VR |  |  | 27 |  |  | 5.3 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | F |  | ----- |  |  |  |  | * 5 |  |  | 150 |  |  |  | FS |  |  |  |  | 50 |  |  |  | P4 |  |  | Bias |  |  |  |  | Volts, 0.4 min., 2.8 max. |
| 1658 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 2.0 |  |  |  |  | 50 |  |  | G |  |  | E |  | 180 |  |  |  |  | 14.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1659 | GM | T |  |  | 16 |  |  | B20 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 1659 | EM | T |  |  | 16 |  |  | 040 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #1. |
| 1659 | EM | T |  |  | 16 |  |  | 030 |  | 50 |  | 2.5 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P2 |  |  | 65 |  |  |  |  | Diode #2. |
| 1662 | GM | T |  |  | 17 |  |  | 423 |  | 00 |  | 2.5 |  |  |  |  | 50 |  |  | J |  |  | C |  | 150 |  |  |  |  | 9.4 |  |  | 90 |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1852 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 6.3 |  |  |  |  | C |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 1853 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 200 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 2050W | TH | VR |  |  | 72 |  |  | 503 |  | 86 |  | 6.3 |  |  |  |  | 5 |  |  | L |  |  | F |  | ----- |  |  |  |  | * 5 |  |  | 135 |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | Bias |  |  |  |  | Volts, 1.5 min., 3.0 max. |
| 5516 | GM | T |  |  | 27 |  |  | 5A3 |  | 00 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 19.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 6.0. |
| 5556 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 4.2 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 20.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.5. |
| 5608A | GM | T |  |  | 17 |  |  | 560 |  | 40 |  | 2.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 5.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5608A | GM | T |  |  | 17 |  |  | 320 |  | 40 |  | 2.5 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 5.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5610 | GM | T |  |  | 34 |  |  | 610 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 90 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5618 | GM | T |  |  | 17 |  |  | 623 |  | 04 |  | 6.3 |  |  |  |  | 50 |  |  | M |  |  | C |  | 175 |  |  |  |  | 10.0 |  |  | 75 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5633 | GM | T |  |  | 46 |  |  | 3A5 |  | 12 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5634 | GM | T |  |  | 46 |  |  | 3A5 |  | 12 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5635 | GM | T |  |  | 36 |  |  | 170 |  | 84 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5635 | GM | T |  |  | 36 |  |  | 250 |  | 84 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5637 | GM | T |  |  | 34 |  |  | 210 |  | 50 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 60 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5640 | GM | T |  |  | 36 |  |  | 157 |  | 20 |  | 6.3 |  |  |  |  | 10 |  |  | N |  |  | C |  | 100 |  |  |  |  | 9.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5644* | VR | VR |  |  | 00 |  |  | 001 |  | 40 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 25 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 105 |  |  |  |  | Max. Max diff -5.0. |
| 5644* | VR | VR |  |  | 00 |  |  | 001 |  | 40 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 85 |  |  |  |  | Min. Max diff -5.0. |
| 5646 | GM | T |  |  | 35 |  |  | 410 |  | 20 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 75 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 83 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5659 | GM | T |  |  | 27 |  |  | 634 |  | 80 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 12.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5660 | GM | T |  |  | 27 |  |  | B36 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | D |  | 225 |  |  |  |  | 3.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 5660 | EM | T |  |  | 27 |  |  | 050 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 5660 | EM | T |  |  | 27 |  |  | 040 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 5661 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | C |  | 225 |  |  |  |  | 3.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 91 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5662 | TH | VR |  |  | 34 |  |  | 107 |  | 50 |  | 6.3 |  |  |  |  | 10 |  |  | L |  |  | F |  | ----- |  |  |  |  | * 10 |  |  | 125 |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | Bias |  |  |  |  | Volts, 0.7 min., 5.7 max. |
| 5675 | GM | T |  |  | 27 |  |  | 350 |  | 60 |  | 6.3 |  |  |  |  | A |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-121. |
| 5679 | EM | T |  |  | 18 |  |  | 060 |  | 75 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 5679 | EM | T |  |  | 18 |  |  | 030 |  | 25 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 5686 | GM | T |  |  | 45 |  |  | 276 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 225 |  |  |  |  | 9.0 |  |  | 225 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 70 |  |  |  |  |  |
| 5694 | GM | T |  |  | 27 |  |  | 430 |  | 10 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5694 | GM | T |  |  | 27 |  |  | 650 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5731 | GM | T |  |  | 16 |  |  | 430 |  | 70 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 7.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5732 | GM | T |  |  | 27 |  |  | B34 |  | 85 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 225 |  |  |  |  | 3.0 |  |  | 125 |  |  |  | RL |  |  |  |  | 67 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5742 | GM | T |  |  | 14 |  |  | 320 |  | 00 |  | 4.2 |  |  |  |  | 10 |  |  | G |  |  | E |  | 250 |  |  |  |  | 8.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 38 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 4.5. |
| 5812 | GM | T |  |  | 34 |  |  | 156 |  | 02 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 27.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 54 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 6.0. |
| 5824 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 26 |  |  |  |  | 50 |  |  | K |  |  | C |  | 125 |  |  |  |  | 22.0 |  |  | 135 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 25.0. |
| 5825 | EM | T |  |  | 14 |  |  | 0A0 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | G |  |  | B |  | 225 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 1.6. |
| 5829 | EM | T |  |  | 35 |  |  | 060 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 75 |  |  |  |  | Diode #1. |
| 5829 | EM | T |  |  | 35 |  |  | 010 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P2 |  |  | 75 |  |  |  |  | Diode #2. |
| 5844 | GM | T |  |  | 34 |  |  | 520 |  | 70 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 80 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5844 | GM | T |  |  | 34 |  |  | 610 |  | 70 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 80 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 56 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5847 | GM | T |  |  | 39 |  |  | 168 |  | 40 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | B |  | 65 |  |  |  |  | CCW |  |  | 60 |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 70 |  |  |  |  |  |
| 5876 | GM | T |  |  | 27 |  |  | 350 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-121. |
| 5881 | GM | T |  |  | 27 |  |  | 534 |  | 81 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 14.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5897 | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 92 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5900 | GM | T |  |  | 36 |  |  | 157 |  | 20 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 5920 | GM | T |  |  | 43 |  |  | 520 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 2.1 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5920 | GM | T |  |  | 43 |  |  | 610 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 2.1 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5965 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | D |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 5965 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | D |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 75 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 5967 | GM | T |  |  | 24 |  |  | 680 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | D |  | 45 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 1.25. |
| 5967 | GM | T |  |  | 75 |  |  | 310 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | Q |  |  | D |  | 45 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 1.25. |
| 6012 | TH | VR |  |  | 27 |  |  | 305 |  | 18 |  | 6.3 |  |  |  |  | 10 |  |  | L |  |  | F |  | ----- |  |  |  |  | * 10 |  |  | 140 |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | Bias |  |  |  |  | Volts, 1.6 min, 4.1 max. |
| 6026 | GM | T |  |  | 45 |  |  | 780 |  | 30 |  | 6.3 |  |  |  |  | D |  |  | M |  |  | B |  | 120 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 81 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6055 | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 26 |  |  |  |  | 5 |  |  | Q |  |  | C |  | 40 |  |  |  |  | 1.4 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 26.5 |
| 6084 | GM | T |  |  | 45 |  |  | 961 |  | 38 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | 100 |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6085 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 5.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6085 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 10 |  |  | G |  |  | C |  | 250 |  |  |  |  | 5.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 63 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6096 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 135 |  |  |  |  | 2.0 |  |  | 120 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6101 | GM | T |  |  | 34 |  |  | 520 |  | 76 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6101 | GM | T |  |  | 34 |  |  | 610 |  | 75 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 89 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6113 | GM | T |  |  | 78 |  |  | 450 |  | 62 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6113 | GM | T |  |  | 78 |  |  | 120 |  | 35 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | D |  | 250 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6134 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 6.3 |  |  |  |  | D |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6137 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 6.3 |  |  |  |  | C |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6140 | VR | VR |  |  | 00 |  |  | 007 |  | 30 |  | OFF |  |  |  |  | 5 |  |  | H |  |  | F |  | 6 |  | MA |  |  | 0.0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 103 |  |  |  |  | Max. Max diff 0.75v. |
| 6140 | VR | VR |  |  | 00 |  |  | 008 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | H |  |  | F |  | 4 |  | MA |  |  | 0.0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 99 |  |  |  |  | Min. Max diff 0.75v. |
| 6148 | GM | T |  |  | 34 |  |  | 712 |  | 65 |  | 6.3 |  |  |  |  | D |  |  | K |  |  | C |  | 135 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 31 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6152 | GM | T |  |  | 45 |  |  | 310 |  | 20 |  | 6.3 |  |  |  |  | E |  |  | L |  |  | C |  | 110 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6159 | GM | T |  |  | 27 |  |  | 5A3 |  | 18 |  | 26 |  |  |  |  | 50 |  |  | K |  |  | B |  | 200 |  |  |  |  | 30.0 |  |  | 200 |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 26.5. |
| 6169 | GM | T |  |  | 36 |  |  | 120 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 120 |  |  |  |  | 0.75 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6197 | GM | T |  |  | 45 |  |  | 168 |  | 17 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 3.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6201 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6201 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6221* | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | B |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6222* | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.6 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6223* | GM | T |  |  | 36 |  |  | 157 |  | 80 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6225* | GM | T |  |  | 36 |  |  | 157 |  | 20 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | C |  | 100 |  |  |  |  | 1.1 |  |  | 100 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6227 | GM | T |  |  | 45 |  |  | 278 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 200 |  |  |  |  | 3.3 |  |  | 200 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6247 | GM | T |  |  | 39 |  |  | 180 |  | 50 |  | 1.1 |  |  |  |  | E |  |  | M |  |  | C |  | 180 |  |  |  |  | ------- |  |  | --------- |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 70 |  |  |  |  |  |
| 6252 | GM | T |  |  | 17 |  |  | 2A3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  | “A” lead to left cap tetrode #1. |
| 6252 | GM | T |  |  | 17 |  |  | 6A3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  | “A” lead to right cap tetrode #2. |
| 6263 | GM | T |  |  | 27 |  |  | 350 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 200 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 6.0. Use Hickock adapter code No. 1050-121. |
| 6264 | GM | T |  |  | 27 |  |  | 350 |  | 60 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | B |  | 200 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 73 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 6.0. Use Hickock adapter code No. 1050-121. |
| 6264 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 44 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6267 | GM | T |  |  | 45 |  |  | 961 |  | 38 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.0 |  |  | 140 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6280 | ------ | ----------- |  |  | ----------------------- |  |  |  |  |  |  | ----------- |  |  |  | ----- |  |  | --------- |  |  | ---------- |  | ------------- |  |  |  |  | ------- |  |  | --------- |  |  | --------- |  |  |  |  | ------ |  |  |  |  | ------- |  |  | ----------- |  |  |  | Special adapter required which is not available and special high-frequency test should be performed. |  |  |
| 6286 | GM | T |  |  | 24 |  |  | 310 |  | 00 |  | 1.1 |  |  |  |  | 5 |  |  | P |  |  | D |  | 70 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 60 |  |  |  |  | Adj fil to 1.25. |
| 6293 | GM | T |  |  | 27 |  |  | 5A3 |  | 60 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | C |  | 150 |  |  |  |  | 30.0 |  |  | 145 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6308 | VR | VR |  |  | 00 |  |  | 003 |  | 10 |  | OFF |  |  |  |  | 5 |  |  | L |  |  | F |  | 1.5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 82 |  |  |  |  | Min. Max diff -3.0. |
| 6308 | VR | VR |  |  | 00 |  |  | 003 |  | 50 |  | OFF |  |  |  |  | 5 |  |  | L |  |  | F |  | 3.5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 92 |  |  |  |  | Max. Max diff -3.0. |
| 6332 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | N |  |  | F |  | 1.5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 63 |  |  |  |  | Max. Max diff -1.0. |
| 6332 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | N |  |  | F |  | .2 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 50 |  |  |  | P5 |  |  | 50 |  |  |  |  | Min. Max diff -1.0. |
| 6350 | GM | T |  |  | 45 |  |  | 860 |  | 70 |  | 12.6 |  |  |  |  | 10 |  |  | M |  |  | C |  | 150 |  |  |  |  | 5.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6350 | GM | T |  |  | 45 |  |  | 310 |  | 20 |  | 12.6 |  |  |  |  | 10 |  |  | M |  |  | C |  | 150 |  |  |  |  | 5.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6354 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  |  |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 116 |  |  |  |  | Max. Max diff -4. |
| 6354 | VR | VR |  |  | 00 |  |  | 001 |  | 20 |  | OFF |  |  |  |  | 5 |  |  | K |  |  | F |  |  |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 107 |  |  |  |  | Min. Max diff -4. |
| 6355 | ER | T |  |  | 35 |  |  | 012 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | F |  | 180 |  |  |  |  | 0 |  |  | 180 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye 1 open, eye 2 closed. |
| 6355 | ER | T |  |  | 35 |  |  | 046 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | F |  | 180 |  |  |  |  | 0 |  |  | 180 |  |  |  | RL |  |  |  |  | 0 |  |  |  | P4 |  |  | ----------- |  |  |  |  | Eye 2 open, eye 1 closed. |
| 6360 | GM | T |  |  | 45 |  |  | 387 |  | 20 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 200 |  |  |  |  | 21.5 |  |  | 200 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode #1. Connect negative (-) end of 30V battery to pin 1 of loctal socket. |
| 6360 | GM | T |  |  | 45 |  |  | 167 |  | 20 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 200 |  |  |  |  | 21.5 |  |  | 200 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode #2. Connect negative (-) end of 30V battery to pin 3 of loctal socket. Connect positive (+) end of 30V battery to pin 2 of loctal socket. Caution: Disconnect battery between selector changes. |
| 6386 | GM | T |  |  | 19 |  |  | 760 |  | 85 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6386 | GM | T |  |  | 19 |  |  | 340 |  | 25 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6414* | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 180 |  |  |  |  | 2.3 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6414* | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 180 |  |  |  |  | 2.3 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6417 | GM | T |  |  | 45 |  |  | 916 |  | 73 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 10.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 41 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6463 | GM | T |  |  | 45 |  |  | 860 |  | 70 |  | 12.6 |  |  |  |  | C |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6463 | GM | T |  |  | 45 |  |  | 310 |  | 20 |  | 12.6 |  |  |  |  | C |  |  | M |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6485 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 6.3 |  |  |  |  | C |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6519 | EM | T |  |  | 53 |  |  | 412 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | Q |  |  | C |  | 50 |  |  |  |  | 1.7 |  |  | 50 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P5 |  |  | 63 |  |  |  |  | Adj fil to 1.25 |
| 6533* | GM | T |  |  | 76 |  |  | 210 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | C |  | 120 |  |  |  |  | 1.7 |  |  | --------- |  |  |  | RL |  |  |  |  | 94 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6540 | GM | T |  |  | 34 |  |  | 712 |  | 65 |  | 6.3 |  |  |  |  | D |  |  | K |  |  | C |  | 135 |  |  |  |  | CCW |  |  | 120 |  |  |  | RL |  |  |  |  | 31 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6542 | VR | VR |  |  | 00 |  |  | 003 |  | 10 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 25 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 125 |  |  |  |  | Max. Max diff 1.25 divisions on quality meter. |
| 6542 | VR | VR |  |  | 00 |  |  | 003 |  | 50 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 25 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 105 |  |  |  |  | Min. Leads are numbered 1, 3, 5, - outside leads are cathode. |
| 6550 | ------ | ----------- |  |  | ----------------------- |  |  |  |  |  |  | ----------- |  |  |  | ----- |  |  | --------- |  |  | ---------- |  | ------------- |  |  |  |  | ------- |  |  | --------- |  |  | --------- |  |  |  |  | ------ |  |  |  |  | ------- |  |  | ----------- |  |  |  | Test requirements beyond tester capabilities. |  |  |
| 6626 | VR | VR |  |  | 00 |  |  | 001 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | G |  |  | F |  | 30 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 126 |  |  |  |  | Max. Max diff -4.5. |
| 6626 | VR | VR |  |  | 00 |  |  | 001 |  | 70 |  | OFF |  |  |  |  | 5 |  |  | H |  |  | F |  | 5 |  | MA |  |  | 0 |  |  | --------- |  |  |  | FS |  |  |  |  | 25 |  |  |  | P5 |  |  | 105 |  |  |  |  | Min. Max diff -4.5. |
| 6660 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 6.3 |  |  |  |  | A |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6661 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6662 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | C |  | 225 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6663 | EM | T |  |  | 34 |  |  | 070 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6663 | EM | T |  |  | 34 |  |  | 020 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6669 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 12.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6677 | GM | T |  |  | 45 |  |  | 263 |  | 17 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 250 |  |  |  |  | 3.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 32 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6678 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 6678 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 6679 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | D |  |  | G |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6679 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | D |  |  | G |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6680 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6680 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6681 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6681 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6686 | GM | T |  |  | 45 |  |  | 278 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | B |  | 210 |  |  |  |  | 3.0 |  |  | 210 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6688 | GM | T |  |  | 45 |  |  | 279 |  | 18 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | A |  | 180 |  |  |  |  | 1.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6689 | GM | T |  |  | 45 |  |  | 261 |  | 39 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 210 |  |  |  |  | 2.1 |  |  | 120 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 6690 | GM | T |  |  | 36 |  |  | 750 |  | 80 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6690 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 6.3 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 40 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6761 | GM | T |  |  | 45 |  |  | 217 |  | 30 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 140 |  |  |  |  | 7.0 |  |  | 120 |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6788 | GM | T |  |  | 36 |  |  | 157 |  | 20 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | D |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6814 | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 6.3 |  |  |  |  | C |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 35 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6832* | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 3.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 6832* | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 3.9 |  |  | --------- |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. |
| 6850 | GM | T |  |  | 53 |  |  | 6A1 |  | 40 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 200 |  |  |  |  | 6.6 |  |  | 200 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  | Cap above octal pins 2 and 3=A. Connect negative (-) end of a 30V battery to pin 2 of octal socket. |
| 6850 | GM | T |  |  | 53 |  |  | 2A1 |  | 40 |  | 12.6 |  |  |  |  | 10 |  |  | K |  |  | C |  | 200 |  |  |  |  | 6.6 |  |  | 200 |  |  |  | RL |  |  |  |  | 43 |  |  |  | P4 |  |  | 63 |  |  |  |  | Cap above octal pins 6 and 7=A. Use Hickock adapter No. 1050-107. Connect negative (-) end of a 30V battery to pin 6 of loctal socket. Connect positive (+) end of a 30V battery to pin 4 of loctal socket for each test. Caution: Disconnect battery (Eveready 413) between selector changes. |
| 6872 | GM | T |  |  | 34 |  |  | 712 |  | 65 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 120 |  |  |  |  | 2.2 |  |  | 120 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6877* | GM | T |  |  | 53 |  |  | 140 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | P |  |  | B |  | 100 |  |  |  |  | 10.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 78 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6887 | EM | T |  |  | 43 |  |  | 070 |  | 10 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | ------ |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6887 | EM | T |  |  | 43 |  |  | 020 |  | 50 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | ------ |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6883 | GM | T |  |  | 27 |  |  | 5A3 |  | 18 |  | 12.6 |  |  |  |  | 50 |  |  | K |  |  | B |  | 200 |  |  |  |  | 30.0 |  |  | 200 |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6888 | GM | T |  |  | 27 |  |  | 486 |  | 53 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | C |  | 150 |  |  |  |  | 4.0 |  |  | 90 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6907 | GM | T |  |  | 17 |  |  | 2A3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  | “A” lead to left cap tetrode #1. |
| 6907 | GM | T |  |  | 17 |  |  | 6A3 |  | 40 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | D |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 34 |  |  |  | P4 |  |  | 65 |  |  |  |  | “A” lead to left cap tetrode #2. |
| 6919 | EM | T |  |  | 34 |  |  | 070 |  | 16 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 6969 | EM | T |  |  | 34 |  |  | 020 |  | 56 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 95 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 6922 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 100 |  |  |  |  | 1.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6922 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 100 |  |  |  |  | 1.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6939* | GM | T |  |  | 54 |  |  | 387 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | H |  |  | D |  | 200 |  |  |  |  | 3.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode #1. Connect negative (-) end of 30V battery to pin 1 of octal socket. Connect positive (+) end of battery to pin 2 of octal socket. |
| 6939* | GM | T |  |  | 54 |  |  | 167 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | H |  |  | D |  | 200 |  |  |  |  | 3.5 |  |  | 150 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 63 |  |  |  |  | Tetrode #2. Connect negative (-) end of 30V battery to pin 3 of octal socket. Connect positive (+) end of battery to pin 2 of octal socket. |
| 6943 | GM | T |  |  | 36 |  |  | 157 |  | 24 |  | 6.3 |  |  |  |  | C |  |  | K |  |  | C |  | 125 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 62 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6944 | GM | T |  |  | 36 |  |  | 157 |  | 24 |  | 6.3 |  |  |  |  | C |  |  | K |  |  | C |  | 125 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6945 | GM | T |  |  | 36 |  |  | 157 |  | 20 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6946 | GM | T |  |  | 36 |  |  | 180 |  | 50 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 6947 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 6.3 |  |  |  |  | E |  |  | M |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6947 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 6.3 |  |  |  |  | E |  |  | M |  |  | C |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 50 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6948 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 6948 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 6.3 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 6954 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 150 |  |  |  |  | 1.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 57 |  |  |  | P4 |  |  | 65 |  |  |  |  | Grid #1. |
| 6954 | GM | T |  |  | 34 |  |  | 756 |  | 21 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | D |  | 150 |  |  |  |  | 1.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Grid #2. |
| 6973 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 15.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7025 | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 67 |  |  |  |  | Triode #1. |
| 7025 | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 67 |  |  |  |  | Triode #2. |
| 7027 | GM | T |  |  | 27 |  |  | 581 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 14.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 75 |  |  |  |  |  |
| 7036 | EM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 26 |  |  |  | P2,P3 |  |  | 45 |  |  |  |  |  |
| 7044 | GM | T |  |  | 45 |  |  | 790 |  | 60 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7044 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7054 | GM | T |  |  | 45 |  |  | 278 |  | 13 |  | 12.6 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 27 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7055 | EM | T |  |  | 34 |  |  | 070 |  | 16 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| 7055 | EM | T |  |  | 34 |  |  | 020 |  | 56 |  | 12.6 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | 0 |  |  |  | RL |  |  |  |  | 66 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| 7056 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 12.6 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 86 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7057 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | D |  |  | K |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7057 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | D |  |  | K |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 68 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7058 | GM | T |  |  | 54 |  |  | 760 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 67 |  |  |  |  | Triode #1. |
| 7058 | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | D |  | 225 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 52 |  |  |  | P4 |  |  | 67 |  |  |  |  | Triode #2. |
| 7059 | GM | T |  |  | 54 |  |  | 263 |  | 70 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | C |  | 200 |  |  |  |  | 1.4 |  |  | 110 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7059 | GM | T |  |  | 54 |  |  | 910 |  | 80 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 150 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7060 | GM | T |  |  | 54 |  |  | 867 |  | 90 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | B |  | 200 |  |  |  |  | 1.5 |  |  | 125 |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7060 | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | G |  |  | C |  | 150 |  |  |  |  | 1.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7061 | GM | T |  |  | 45 |  |  | 391 |  | 70 |  | 12.6 |  |  |  |  | 52 |  |  | G |  |  | C |  | 250 |  |  |  |  | 12.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7077 | ------ | ----------- |  |  | ----------------------- |  |  |  |  |  |  | ----------- |  |  |  | ----- |  |  | --------- |  |  | ---------- |  | ------------- |  |  |  |  | ------- |  |  | --------- |  |  | --------- |  |  |  |  | ------ |  |  |  |  | ------- |  |  | ----------- |  |  |  | Special adapter required; not available. |  |  |
| 7119 | GM | T |  |  | 54 |  |  | 790 |  | 60 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 23 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #1. |
| 7119 | GM | T |  |  | 54 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 23 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode #2. |
| 7167 | GM | T |  |  | 34 |  |  | 156 |  | 20 |  | 12.6 |  |  |  |  | 5 |  |  | L |  |  | B |  | 125 |  |  |  |  | 1.0 |  |  | 80 |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 13.5. |
| 7189 | GM | T |  |  | 45 |  |  | 279 |  | 30 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 29 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7199 | GM | T |  |  | 54 |  |  | 723 |  | 60 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | B |  | 220 |  |  |  |  | 1.5 |  |  | 130 |  |  |  | RL |  |  |  |  | 71 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7199 | GM | T |  |  | 54 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | 10 |  |  | J |  |  | C |  | 200 |  |  |  |  | 7.4 |  |  | --------- |  |  |  | RL |  |  |  |  | 74 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7247 | ------ | ----------- |  |  | ----------------------- |  |  |  |  |  |  | ----------- |  |  |  | ----- |  |  | --------- |  |  | ---------- |  | ------------- |  |  |  |  | ------- |  |  | --------- |  |  | --------- |  |  |  |  | ------ |  |  |  |  | ------- |  |  | ----------- |  |  |  |  |  |  |
| 7258 | GM | T |  |  | 45 |  |  | 867 |  | 91 |  | 12.6 |  |  |  |  | 5 |  |  | L |  |  | B |  | 160 |  |  |  |  | 0.9 |  |  | 125 |  |  |  | RL |  |  |  |  | 61 |  |  |  | P4 |  |  | 63 |  |  |  |  | Pentode sect. |
| 7258 | GM | T |  |  | 45 |  |  | 217 |  | 36 |  | 12.6 |  |  |  |  | 5 |  |  | L |  |  | D |  | 150 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 16 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 7308 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 6.3 |  |  |  |  | 5 |  |  | N |  |  | B |  | 100 |  |  |  |  | 1.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7308 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 6.3 |  |  |  |  | ----- |  |  | N |  |  | B |  | 100 |  |  |  |  | 1.2 |  |  | --------- |  |  |  | RL |  |  |  |  | 25 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7316 | GM | T |  |  | 45 |  |  | 760 |  | 80 |  | 12.3 |  |  |  |  | 10 |  |  | J |  |  | D |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7316 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.3 |  |  |  |  | 10 |  |  | J |  |  | D |  | 250 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7355 | GM | T |  |  | 72 |  |  | 638 |  | 50 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 15.0 |  |  | 225 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 63 |  |  |  |  |  |
| 7360 | GM | T |  |  | 45 |  |  | 362 |  | 19 |  | 6.3 |  |  |  |  | 5 |  |  | K |  |  | C |  | 150 |  |  |  |  | 1.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 28 |  |  |  | P4 |  |  | 60 |  |  |  |  | Connect pin 1 to pin 8 and pin 6 to pin 7 on loctal socket. |
| 7370 | GM | T |  |  | 58 |  |  | 790 |  | 60 |  | 20.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 29 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7370 | GM | T |  |  | 84 |  |  | 210 |  | 30 |  | 20.0 |  |  |  |  | 5 |  |  | M |  |  | B |  | 120 |  |  |  |  | 2.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 29 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7408 | GM | T |  |  | 72 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 12.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 48 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7486 | ------ | ----------- |  |  | ----------------------- |  |  |  |  |  |  | ----------- |  |  |  | ----- |  |  | --------- |  |  | ---------- |  | ------------- |  |  |  |  | ------- |  |  | --------- |  |  | --------- |  |  |  |  | ------ |  |  |  |  | ------- |  |  | ----------- |  |  |  | Special adapter reqiored; not available. |  |  |
| 7543 | GM | T |  |  | 34 |  |  | 156 |  | 72 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 125 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7551 | GM | T |  |  | 45 |  |  | 263 |  | 17 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 18.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 99 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 13.5. |
| 7558 | GM | T |  |  | 45 |  |  | 263 |  | 17 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 18.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 99 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7581 | GM | T |  |  | 27 |  |  | 534 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 14 |  |  | 250 |  |  |  | RL |  |  |  |  | 23 |  |  |  | P4 |  |  | 47 |  |  |  |  |  |
| 7586 | GM | T |  |  | 31 |  |  | 420 |  | 80 |  | 6.3 |  |  |  |  | 5 |  |  | Q |  |  | B |  | 40 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 31 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-127. |
| 7587 | GM | T |  |  | 31 |  |  | 4A2 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | M |  |  | B |  | 125 |  |  |  |  | CCW |  |  | 50 |  |  |  | RL |  |  |  |  | 32 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-127. |
| 7591 | GM | T |  |  | 27 |  |  | 638 |  | 50 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | B |  | 250 |  |  |  |  | 10 |  |  | 250 |  |  |  | RL |  |  |  |  | 55 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7687 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | J |  |  | C |  | 220 |  |  |  |  | .8 |  |  | 130 |  |  |  | RL |  |  |  |  | 30 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7687 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | C |  | 215 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 77 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7693 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | B |  |  | J |  |  | C |  | 250 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 46 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7694 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | B |  |  | M |  |  | C |  | 225 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 60 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7695 | GM | T |  |  | 45 |  |  | 691 |  | 70 |  | 50.0 |  |  |  |  | 50 |  |  | K |  |  | B |  | 130 |  |  |  |  | 11 |  |  | 130 |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7701 | GM | T |  |  | 45 |  |  | 269 |  | 10 |  | 14.0 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 12.5 |  |  | 250 |  |  |  | RL |  |  |  |  | 65 |  |  |  | P4 |  |  | 65 |  |  |  |  | Adj fil to 13.6. |
| 7716 | GM | T |  |  | 45 |  |  | 798 |  | 60 |  | 12.6 |  |  |  |  | B |  |  | J |  |  | B |  | 200 |  |  |  |  | 0 |  |  | 125 |  |  |  | RL |  |  |  |  | 42 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7716 | GM | T |  |  | 45 |  |  | 230 |  | 10 |  | 12.6 |  |  |  |  | 5 |  |  | M |  |  | C |  | 125 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 69 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7719 | GM | T |  |  | 45 |  |  | 210 |  | 30 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 250 |  |  |  |  | 10.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 58 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7724 | GM | T |  |  | 45 |  |  | 890 |  | 70 |  | 14.0 |  |  |  |  | 5 |  |  | J |  |  | D |  | 250 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 97 |  |  |  | P4 |  |  | 63 |  |  |  |  | Triode sect. |
| 7724 | EM | T |  |  | 45 |  |  | 020 |  | 30 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #1. |
| 7724 | EM | T |  |  | 45 |  |  | 060 |  | 10 |  | 14.0 |  |  |  |  | 5 |  |  | S |  |  | A |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 100 |  |  |  | P2 |  |  | 40 |  |  |  |  | Diode #2. |
| 7728 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | D |  |  | G |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7728 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | D |  |  | G |  |  | B |  | 250 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 87 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7729 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7729 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 82 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7730 | GM | T |  |  | 45 |  |  | 760 |  | 81 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7730 | GM | T |  |  | 45 |  |  | 210 |  | 36 |  | 12.6 |  |  |  |  | 10 |  |  | J |  |  | C |  | 225 |  |  |  |  | 8.5 |  |  | --------- |  |  |  | RL |  |  |  |  | 80 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7731 | GM | T |  |  | 45 |  |  | 263 |  | 70 |  | 6.3 |  |  |  |  | A |  |  | J |  |  | B |  | 250 |  |  |  |  | CCW |  |  | 135 |  |  |  | RL |  |  |  |  | 95 |  |  |  | P4 |  |  | 65 |  |  |  |  | Pentode sect. |
| 7731 | GM | T |  |  | 45 |  |  | 910 |  | 80 |  | 6.3 |  |  |  |  | A |  |  | G |  |  | B |  | 150 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 53 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| 7732 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | D |  |  | H |  |  | B |  | 200 |  |  |  |  | CCW |  |  | 150 |  |  |  | RL |  |  |  |  | 88 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7733 | GM | T |  |  | 46 |  |  | 278 |  | 13 |  | 12.6 |  |  |  |  | 5 |  |  | J |  |  | C |  | 250 |  |  |  |  | 2.6 |  |  | 140 |  |  |  | RL |  |  |  |  | 37 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7737 | GM | T |  |  | 45 |  |  | 279 |  | 18 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | A |  | 180 |  |  |  |  | 1.0 |  |  | 150 |  |  |  | RL |  |  |  |  | 90 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7754 | GM | T |  |  | 45 |  |  | 691 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | K |  |  | B |  | 130 |  |  |  |  | 11 |  |  | 130 |  |  |  | RL |  |  |  |  | 24 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7759 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 26.0 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 39 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7759 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 26.0 |  |  |  |  | B |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 39 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7760 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 26.0 |  |  |  |  | 5 |  |  | Q |  |  | B |  | 40 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7760 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 26.0 |  |  |  |  | 5 |  |  | Q |  |  | B |  | 40 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7761 | GM | T |  |  | 36 |  |  | 157 |  | 24 |  | 26.0 |  |  |  |  | B |  |  | L |  |  | B |  | 150 |  |  |  |  | CCW |  |  | 100 |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7762 | GM | T |  |  | 36 |  |  | 157 |  | 42 |  | 26.0 |  |  |  |  | E |  |  | N |  |  | C |  | 110 |  |  |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 47 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |
| 7868 | GM | T |  |  | 12 |  |  | 356 |  | 78 |  | 6.3 |  |  |  |  | 10 |  |  | G |  |  | B |  | 250 |  |  |  |  | 7.0 |  |  | 250 |  |  |  | RL |  |  |  |  | 33 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-144. |
| 7889 | GM | T |  |  | 36 |  |  | 780 |  | 50 |  | 25.0 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| 7889 | GM | T |  |  | 36 |  |  | 210 |  | 40 |  | 25.0 |  |  |  |  | E |  |  | N |  |  | C |  | 100 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 76 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| 7895 | GM | T |  |  | 13 |  |  | 420 |  | 80 |  | 6.3 |  |  |  |  | D |  |  | N |  |  | B |  | 110 |  |  |  |  | CCW |  |  | --------- |  |  |  | RL |  |  |  |  | 45 |  |  |  | P4 |  |  | 65 |  |  |  |  | Use Hickock adapter code No. 1050-127. |
| 9003 | GM | T |  |  | 34 |  |  | 156 |  | 27 |  | 6.3 |  |  |  |  | 5 |  |  | M |  |  | D |  | 175 |  |  |  |  | 2.4 |  |  | 80 |  |  |  | RL |  |  |  |  | 59 |  |  |  | P4 |  |  | 64 |  |  |  |  | Short in Z. |
| XXB | GM | T |  |  | 78 |  |  | 360 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 98 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. Adj fil to 1.4. |
| XXB | GM | T |  |  | 17 |  |  | 430 |  | 00 |  | 1.5 |  |  |  |  | 5 |  |  | N |  |  | D |  | 100 |  |  |  |  | 3.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 98 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. Adj fil to 1.4. |
| XXD | GM | T |  |  | 18 |  |  | 560 |  | 70 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 10.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #1. |
| XXD | GM | T |  |  | 18 |  |  | 430 |  | 20 |  | 12.6 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 10.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 85 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode #2. |
| XXFM | GM | T |  |  | 18 |  |  | 320 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | G |  |  | C |  | 250 |  |  |  |  | 1.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 97 |  |  |  | P4 |  |  | 65 |  |  |  |  | Triode sect. |
| XXFM | EM | T |  |  | 18 |  |  | 050 |  | 40 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #1. |
| XXFM | GM | T |  |  | 18 |  |  | 060 |  | 70 |  | 6.3 |  |  |  |  | 5 |  |  | S |  |  | B |  | 20 |  | AC |  |  | 0 |  |  | --------- |  |  |  | RL |  |  |  |  | 21 |  |  |  | P2 |  |  | 50 |  |  |  |  | Diode #2. |
| XXL | GM | T |  |  | 18 |  |  | 620 |  | 70 |  | 6.3 |  |  |  |  | 50 |  |  | G |  |  | C |  | 250 |  |  |  |  | 8.0 |  |  | --------- |  |  |  | RL |  |  |  |  | 70 |  |  |  | P4 |  |  | 65 |  |  |  |  |  |

| Ballast tube type | FIL-selector switch position | FIL+selector switch positions (FIL CONT- SHORT lamp shold glow in each position) |
| --- | --- | --- |
| 1A1/5E1 | 1 | 4 |
| 1B1 | 1 | 4 |
| 1C1 | 1 | 4 |
| 1E1 | 1 | 4 |
| 1F1 | 1 | 4 |
| 1G1 | 1 | 4 |
| 1J1 | 1 | 4 |
| 1K1 | 1 | 4 |
| 1L1 | 2 | 7 |
| 1N1 | 2 | 7 |
| 1P1 | 2 | 7 |
| 1Q1 | 2 | 7 |
| 1R1G | 2 | 7 |
| 1S1G | 2 | 7 |
| 1T1G | 2 | 7 |
| 1U1G | 2 | 7 |
| 1V1 | 1 | 4 |
| 1X1 | 1 | 4 |
| 1Y1 | 1 | 4 |
| 1Z1 | 1 | 4 |
| 2 | 1 | 4 |
| 2UR224 | 3 7 | 7,8 8 |
| 2LR212 | 4 | 1,2,8 |
| O3G | 3 | 7 |
| 3 | 1 | 4 |
| 4 | 1 | 4 |
| 5 | 1 | 4 |
| 6-133 | 3 | 7 |
| 6 | 1 | 4 |
| 6-6AA | 2 | 7 |
| 7 | 1 | 4 |
| 8 | 1 | 4 |
| 9 | 1 | 4 |
| 10A | 3 | 7 |
| 10AG | 3 | 7 |
| 10AB | 3 | 7,8 |
| K17B | 3 | 7,8 |
| M17C | 3 | 7,8 |
| BM17C | 3 | 7,8 |
| M17H7, M17H | 1 3 | 7,8 3 |
| L23P | 3 | 7,8 |
| K23C | 1 | 7,8 |
| KX23B | 1 | 3,4 |
| M30H | 2 1 | 3 7,8 |
| 30A, K30A | 3 | 7 |
| K30D | 3 | 2,7,8 |
| 33A, 33AG | 3 | 7 |
| K34B | 3 | 7,8 |
| 36A | 3 | 7 |
| K36B, BK36B | 3 | 7,8 |
| L36B | 3 | 7,8 |
| L36C | 3 | 7,8 |
| KX36C | 1 | 3,4 |
| KX36A | 1 | 4 |
| 36D | 3 | 2,7,8 |
| L36D | 3 | 2,7,8 |
| L36DJ | 4 | 2,7,8 |
| K36H | 2 1 | 3 7,8 |
| M36H, M36HG | 2 1 | 3 7,8 |
| L40S1 | 3 | 2,5,7 |
| L40S2 | 3 | 2,5,7 |
| 42A | 3 | 7 |
| 42A1 | 4 | 8 |
| 42A2 | 4 | 1,8 |
| 42B2 | 4 | 1,8 |
| K42B | 3 | 7,8 |
| L42B | 3 | 7,8 |
| M42B | 3 | 7,8 |
| KX42B | 1 | 3,4 |
| LX42B | 1 | 3,4 |
| L42BX | 3 | 7,8 |
| K42C | 3 | 7,8 |
| L42C | 3 | 7,8 |
| M42C | 3 | 7,8 |
| KB42D | 3 | 2,7,8 |
| K42D | 3 | 2,7,8 |
| L42D | 3 | 2,7,8 |
| LX42D | 1 | 2,3,4 |
| L42DX | 1 | 2,3,4 |
| K42E | 3 | 2,5,7,8 |
| L42E | 3 | 2,5,7,8 |
| L42F | 2 7 | 3 8 |
| 42HA | 1 2 | 7,8 3,4 |
| K42HJ | 4 1 | 2 7,8 |
| M42H, M32HG | 2 1 | 3 7,8 |
| KX42C | 1 | 3,4 |
| L42S1 | 3 | 5,6 |
| 49A | 3 | 7 |
| 49AJ, K49AJ | 4 | 7 |
| KX49A | 1 | 4 |
| 49A1 | 4 | 8 |
| 49A2 | 4 | 1,8 |
| 49B2 | 4 | 1,8 |
| K49B | 3 | 7,8 |
| M49B, BM49B | 3 | 7,8 |
| K47C | 3 | 7,8 |
| M49C, BM49C | 3 | 7,8 |
| BK49C | 3 | 7,8 |
| K49E | 3 | 2,5,7,8 |
| L49E | 3 | 2,5,7,8 |
| K49D, BK49D | 3 | 2,7,8 |
| L49D | 3 | 2,7,8 |
| L49F | 2 7 | 3 8 |
| M49H, M49HG | 2 1 | 3 7,8 |
| KZ59B? | 2 | 5,7 |
| KZ49C | 2 | 5,7 |
| K49BJ | 4 | 7,8 |
| L49PJ | 4 | 7,8 |
| L49S2 | 3 | 2,5,7 |
| 49AJ, K49AJ | 4 | 7 |
| KX49B | 1 | 3,4 |
| LX49B | 1 | 3,4 |
| LX49X | 1 | 3,4 |
| L49DJ | 4 | 2,7,8 |
| L49S3 | 3 | 2,5,7 |
| 50A2 | 1 | 3,4 |
| 50A2MG | 2 | 3,4 |
| 50B2 | 2 | 5,7 |
| 50X3 | 1 | 4 |
| K52H | 2 1 | 3 7,8 |
| M52H | 2 1 | 3 7,8 |
| K54B | 3 | 7,8 |
| 55A, K55A | 3 | 7 |
| 55A1 | 4 | 8 |
| KX55A | 1 | 4 |
| 55B, K55B | 3 | 7,8 |
| M55B, BM55B | 3 | 7,8 |
| L55BG | 3 | 7,8 |
| LX55B | 1 | 3,4 |
| 55A2 | 4 | 1,8 |
| 55B2 | 4 | 1,8 |
| K55C | 3 | 7,8 |
| L55C | 3 | 7,8 |
| KX55C | 1 | 3,4 |

|  |  |  |  |  |  | TB 11-6625-316-12/1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | HAROLD K. JOHNSON, |  |  |
|  |  |  |  |  |  | General, United States Army, |  |  |
| Official: |  |  |  |  |  | Chief of Staff. |  |  |
|  | J. C. LAMBERT, |  |  |  |  |  |  |  |
|  | Major General, United States Army, |  |  |  |  |  |  |  |
|  | he Adjutant General. |  |  |  |  |  |  |  |
| Distribution: |  |  |  |  |  |  |  |  |
|  |  | Active Army: |  |  |  |  |  |  |
|  |  |  | USASA (2) |  | USASTRATCOM (4) |  |  |  |
|  |  |  | CNGB (1) |  | MDW (1) |  |  |  |
|  |  |  | CC-E (7) |  | Armies (2) except |  |  |  |
|  |  |  | Dir of Trans (1) |  |  | Third (5) |  |  |
|  |  |  | CofEngrs (1) |  |  | Eighth (5) |  |  |
|  |  |  | TSG (1) |  | Corps (2) |  |  |  |
|  |  |  | CofSptS (1) |  | USAC (3) |  |  |  |
|  |  |  | USAAVNTBD (5) |  | 507th USASA Gp (5) |  |  |  |
|  |  |  | USACDCEA (1) |  | 318th USASA Bn (5) |  |  |  |
|  |  |  | USACDCCBRA (1) |  | 319th USASA Bn (5) |  |  |  |
|  |  |  | USACDCCEA (1) |  | 320th USASA Bn (5) |  |  |  |
|  |  |  | USACDCCEA |  | 1st USASA Fld Sta (5) |  |  |  |
|  |  |  |  | Ft Huachuca (1) | 2nd USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCOA (1) |  | 3rd USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCQMA (1) |  | 4th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCTA (1) |  | 5th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCADA (1) |  | 9th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCARMA (1) |  | 12th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCAVNA (1) |  | 13th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCARTYA (1) |  | 14th USASA Fld Sta (5) |  |  |  |
|  |  |  | USACDCSWA (1) |  | 15th USASA Fld Sta (5) |  |  |  |
|  |  |  | USAMC (5) |  | Svc Colleges (2) |  |  |  |
|  |  |  | USCONARC (5) |  | Br Svc Sch (2) except |  |  |  |
|  |  |  | ARADCOM (5) |  |  | USATSCG (5) |  |  |
|  |  |  | ARADCOM Rgn (2) |  |  | USACSS (5) |  |  |
|  |  |  | OS Maj Comd (4) |  |  | USAAVNS (5) |  |  |
|  |  |  | USASTRATCOM-SEA (5) |  |  | USASCS (60) |  |  |
|  |  |  | USAREUR (5) |  |  | USASESCS (60) |  |  |
|  |  |  | LOGCOMD (2) |  |  | USA Msl & Mun Cen |  |  |
|  |  |  | USAMICOM (4) |  |  |  | & Sch (60) |  |
|  |  |  | USAMC (2) |  |  | USAADS (60 |  |  |
|  |  |  | USATC AD (2) |  | GENDEP (OS) (2) |  |  |  |
|  |  |  | USATC Armor (2) |  | Sig Sec, GENDEP (OS) (5) |  |  |  |
|  |  |  | USATC Engr (2) |  | Sig Dep (OS) (12) |  |  |  |
|  |  |  | USATC Inf (2) |  | Sig Fld Maint Shops (2) |  |  |  |
|  |  |  | USASTC (2) |  | AMS (1) |  |  |  |
|  |  |  | WRAMC (1) |  | USAERDAA (2) |  |  |  |
|  |  |  | Army Pic Cen (2) |  | USAERDAW (13) |  |  |  |
|  |  |  | USAINTC (5) |  | USACRREL (2) |  |  |  |
|  |  |  | Inst (2) except |  | Units organized under following TOE’s: |  |  |  |
|  |  |  |  | Ft Monmouth (70) |  | 6-526 (2) |  | 29-21 (2) |
|  |  |  |  | Ft Hancock (4) |  | 7 (2) |  | 29-25 (2) |
|  |  |  |  | Ft Gordon (10) |  | 7-100 (2) |  | 29-26 (2) |
|  |  |  |  | Ft Huachuca (10 |  | 11-6 (2) |  | 29-35 (2) |
|  |  |  |  | WSMR (5) |  | 11-56 (2) |  | 29-36 (2) |
|  |  |  |  | Ft Carson (19) |  | 11-57 (2) |  | 29-51 (2) |
|  |  |  |  | Ft Knox (12) |  | 11-97 (2) |  | 29-55 (2) |
|  |  |  |  | JCA, Ft Ritchie (5) |  | 11-98 (2) |  | 29-56 (2) |
|  |  |  |  | Ft Devans (5) |  | 11-117 (2) |  | 29-75 (2) |
|  |  |  |  | Edgewood Arsenal (5) |  | 11-127 (2) |  | 29-79 (2) |
|  |  |  |  | USAEPG (5) |  | 11-155 (2) |  | 29-105 (2) |
|  |  |  |  | APG (5) |  | 11-157 (2) |  | 29-109 (2) |
|  |  |  | USACA, Taiwan (3) |  |  | 11-158 (2) |  | 30-25 (2) |
|  |  |  | Army Dep (2) except |  |  | 11-500 AA-AC (2) |  | 30-26 (2) |
|  |  |  |  | LBAD (14) |  | 11-587 (2) |  | 30-29 (2) |
|  |  |  |  | SAAD (30) |  | 11-592 (2) |  | 37 (2) |
|  |  |  |  | TOAD (14) |  | 11-597 (2) |  | 37-100 (2) |
|  |  |  |  | FTWOAD (10) |  | 17 (2) |  | 39-51 (2) |
|  |  |  |  | LEAD (7) |  | 17-100 (2) |  | 39-52 (2) |
|  |  |  |  | SHAD (3) |  | 29-1 (2) |  | 55-157 (2) |
|  |  |  |  | NAAD (5) |  | 29-15 (2) |  | 57 (2) |
|  |  |  |  | SVAD (5) |  | 29-16 (2) |  | 57-100 (2) |
|  |  |  |  | CHAD (3) |  |  |  |  |
|  |  |  |  | ATAD (10) |  |  |  |  |
|  |  | NG: State AG (3) |  |  |  |  |  |  |
|  |  | USAR: None. |  |  |  |  |  |  |
|  |  | For explanation of abbreviations used, see AR 320-50. |  |  |  |  |  |  |

* This bulletin supersedes TB 11-2661-1,  27 January 1960, including C 1, 26 July 1960; C 2, 30 March 1961; C 3, 26 October 1961; and C 4, 1 January 1962.

# Adjust plate and screen controls so that plate meter reads 3.0 MA.

# Adjust plate and screen controls so that plate meter reads 3.0 MA.

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)

* OK under 100 on percent quality meter.  (Tube voltage drop measured.)